This  international  standard was developed in  accordance with internationally recognized principles on standardization established in the Decision on Principles for the Development of International Standards,Guides and Recommendations issued by the World Trade Organization Technical Barriers to Trade(TBT)Committee.

Designation:F3411-22a

Standard Specification for

Remote ID and Tracking¹

This  standard  is  issued  under  the fixed designation  F3411;the  number immediately  following  the  designation  indicates  the  year of original  adoption  or,in  the  case  of  revision,the  year  of  last  revision.A  number  in  parentheses  indicates  the  year  of  last  reapproval.A superscript  epsilon(e)indicates  an  editorial  change since  the  last  revision  or  reapproval

NOTE—Subsection  5.4.3  and  Table  A3.3 were  corrected,with  other editorial changes made throughout,and the yeardate changed on June  17.2022.

1.Scope

1.1 This specification covers the performance requirements for  remote  identification(Remote ID)of unmanned aircraft systems(UAS).Remote ID allows governmental and civil identification of UAS for safety,security,and compliance purposes.The objective is to increase UAS remote pilot accountability by removing anonymity while preserving operational privacy for remote pilots,businesses,and their customers.Remote ID is an enabler of enhanced operations such as beyond  visual  line of sight(BVLOS)operations as well  as operations over people.

1.2  This  specification  defines  message formats,transmission  methods,and  minimum  performance  standards  for  two forms  of  Remote  ID:broadcast  and  network.Broadcast  Re- mote ID is based on the transmission of radio signals directly from  a  UAS  to  receivers  in  the UAS's vicinity.Network Remote ID is based on communication by means of the internet from a network Remote ID service provider(Net-RID SP)that interfaces directly or indirectly with the UAS,or with other sources in the case of intent-based network participants.

1.3  This  specification  addresses  the  communications  and  test requirements of broadcast or network Remote ID,or both, in UAS and Net-RID SP systems.

1.4  Applicability:

1.4.1 This specification is applicable to UAS that operate at very low level(VLL)airspace  over diverse environments including  but  not  limited  to  rural,urban,networked,network degraded,and network denied environments,regardless of airspace class.

1.4.2  This  specification  neither  purports  to  address UAS operating with approval to use ADS-B or secondary surveillance radar transponders,nor does it purport to solve ID needs of UAS for all operations.

!This specification  is under the jurisdiction  of ASTM  Committee  F38 on  Unmanned Aircraft  Systems  and  is  the  direct  responsibility  of Subcommittee F38.02 on  Flight Operations.Current edition approved  June 17,2022.Published  July 2022.Originally approved in  2019.Last previous  edition  approved in  2022 as  F3411-22.DOI: 10.1520/F3411-22A.

1.4.3  In  particular,this specification  does  not  purport  to address identification needs for UAS that are not participating in  Remote  ID  or  operators  that  purposefully  circumvent Remote  ID.

1.5  The  values  stated  in  SI  units  are  to  be  regarded  as standard.The  values  given  in  parentheses  after  SI  units  are provided for information only and are not considered standard.

1.5.1 Units of measurement included in this specification:

m meters

direction

deg,° degrees of latitude and  longitude,compass

s  seconds

Hz  Hertz(frequency)

dBm  decibel-milliwatts(radio  frequency  power) ppm parts  per million(radio frequency variation) μs microseconds

ms milliseconds

1.6 Table of Contents:

Title

ecfio

Scope

2

3

4

5

6

7

8

9

10

11

12

13

14

Annex A1  Annex A2

Annex A3  Annex A4

Annex A5

Appendix X1

Appendix X2

Appendix  ×3

Referenced Documents Terminology

Remote ID and Network Interoperability Conceptual Overview Performance Requirements

TEST METHODS Scope

Significance and Use Hazards

Test  Units

Procedure

Precision and Bias Product Marking

Packaging and Package Marking

Keywords

ANNEX A1—Broadcast Authentication Verifier Service

ANNEX A2—Network Remote ID Interoperability Requirements,

APls,and  Testing

ANNEX A3—Tables of Values

ANNEX A4—USS-DSS and USS-USS OpenAPI YAML Description

ANNEX A5—Number Registrar Management Policy

APPENDIX X1—Performance Characteristics

APPENDIX X2—List of Subcommittee Participants and

Contributors

APPENDIX×3—Background Information

Copyright ◎ASTM International,100 Barr Harbor Drive,PO Box C700,West Conshohocken,PA 19428-2959.United States

1.7  This standard does not purport  to address all of the safety concerns,if any,associated with its  use.It is  the responsibility of the user of this standard to establish appropriate  safety,health,and  environmental practices  and  deter- mine  the  applicability  of  regulatory limitations  prior to use.Some specific hazards statements are given in Section 8 on  Hazards.

1.8  This  international  standard  was  developed  in  accordance with internationally recognized principles on standard- ization  established  in the  Decision on  Principles  for  the Development of International Standards,Guides and Recommendations issued by the World Trade Organization Technical Barriers  to Trade(TBT)Committee.

2.Referenced Documents

2.1 ASTM Standards:²

F3060 Terminology for Aircraft

F3341 Terminology for Unmanned Aircraft Systems

2.2  Other Standards:

ANSI/CTA-2063-A Small Unmanned Aerial Systems Serial Numbers³

Bluetooth4,5Core Specification 5.0⁶

IEEE 802.11  Standard  for  Information  technology-- Telecommunications and information exchange between systems -Local and metropolitan area networks--Specific requirements  -Part  11:Wireless  LAN  Medium Access  Control(MAC)and  Physical  Layer(PHY) Specifications7,5

IEEE 1609.2 IEEE Standard for Wireless Access in Vehicu- lar Environments--Security Services for Applications and Management Messages⁷

IETF RFC3339 Date and Time on the Internet:Timestamps⁸

IETF  RFC4122  A  Universally  Unique  IDentifier(UUID) URN Namespace⁹

IETF RFC8126 Guidelines for Writing an IANA Considerations Section in RFCs₁0

Neighbor Awareness Networking Specification11.5

FAA UTM ConOps v1.0 Unmanned  Aircraft  System(UAS) Traffic Management (UTM)Concept of Operations¹²

WGS-84 World  Geodetic  System—1984¹ ³

²For  referenced  ASTM  standards,visit  the  ASTM  website,www.astm.org,or contact ASTM Customer Service at service@astm.org.For Annual Book of ASTM Standards volume information,refer to the standard's Document Summary page on  the ASTM website.

3 Available from American National Standards Institute(ANSI),25 W.43rd St., 4th  Floor,New  York,NY 10036,http://www.ansi.org.

4Used throughout  the  specification,Bluetooth is  a  registered trademark  of Bluetooth SIG,Inc.,5209 Lake  Washington Blvd.NE,Suite  350,Kirkland,WA 98033.

5Other names and brands may be claimed as the property of others.

⁶Available from https://www.bluetooth.com/specifications/archived- %20specifications/.

⁷Available  from  Institute  of  Electrical  and  Electronics  Engineers,Inc.(IEEE), 445  Hoes Ln.,Piscataway,NJ 08854-4141,https://standards.ieee.org/standard/802 _11-2016.html.

8Available  from  IETF  Tools,https:/tools.ietf.org/html/rfc3339.

9 Available from IETF Tools,https:/tools.ietf.org/html/rfc4122.

10 Available from https://datatracker.ief.org/doc/html/rfc8126.

IAvailable from Wi-Fi Alliance,10900-B Stonelake Boulevard,Suite  126, Austin,TX 78759,https://www.wi-fi.org/discover-wi-fi/wi-fi-aware.

1²Available  from  https:/utm.arc.nasa.gov/docs/2018-UTM-ConOps-v1.0.pdf.

13 Available from International Civil Aviation Organization (ICAO),999 Robert- Bourassa  Boulevard,Montréal,Quebec,Canada  H3C  5H7,https://www.icao.int/  safety/pbn/Documentation/EUROCONTROL/Eurocontrol%20WGS%2084%20Implementation%20Manual.pdf.

3.Terminology

3.1 This standard uses terminology contained within F3341, UAS Terminology Standard,and F3060,Aircraft  Terminology Standard.These terms are not duplicated within this document.

3.2  Unique and Common Terminology—Terminology used in multiple standards is defined in F3341 and F3060. Termi- nology that is unique to this specification is defined in 3.3.

3.3 Definitions of Terms Specific to This Standard:

3.3.1  authentication,n—the process  or  action  of verifying that the source of a Remote ID message is the originator of the message.

3.3.2 broadcast,v—to transmit data to no specific destina- tion  or  recipient;data  can  be  received  by  anyone  within broadcast range.

3.3.3 broadcast UAS,n—a UAS that is equipped for and is actively  broadcasting  Remote  ID  data  during  an  operation; being a broadcast UAS is not mutually exclusive with being a networked  UAS.

3.3.4  discovery,n—the  process  of  determining  the  set  of USSs with which data exchange is required for some UTM function;discovery is accomplished by means of the discovery and  synchronization  service(DSS).

3.3.5 DSS entity,n—a generic concept that refers to infor- mation that can be discovered using the discovery and syn- chronization service(DSS).

3.3.5.1  Discussion—Entities  are  characterized  by  a  4-D volume of airspace(that is,a volume defined in x,y,z plus time limits).For  Remote  ID,the  entity  type  is  referred  to  as  an identification service area.Operations and constraints are examples of other types of entities that are the subject of other UTM standards.

3.3.6  DSS  pool,n—a  synchronized  set  of  DSS  instances where operations may be performed on any instance with the same result,and information may be queried from any instance with the same result.A DSS region will often have a produc- tion DSS pool along with one or more test or staging DSS pools.

3.3.7  DSS  region,n—the  geographic  area  supported  by  a DSS pool.

3.3.8 dynamic data,n—data that changes over the duration of the  flight;for  example,longitude  and  latitude.

3.3.9 Ground Control Station(GCS),n—the part of a  UAS that remotely controls the UA.It may or may not have a remote pilot directly manipulating the controls.

3.3.10  identify—the  result of the process  to  establish  the identity of a specific UAS that is traceable to the owner and remote pilot.

3.3.11 intent-based network  participant,n—a UAS  for which the operator has reported an intended area(a volume ofairspace)and time for an operation through a Net-RID service provider;such  information  is  then  reported  through  the  net- work Remote ID infrastructure.Intent-based Remote ID par- ticipation is an option for non-equipped UAS or UAS operating in environments that preclude broadcast or network participa- tion.

3.3.12  network Remote ID(Net-RID)service provider;n—a logical  entity  denoting  a  UTM  system  or  comparable  UAS flight management system that participates in network Remote ID and provides data for and about UAS it manages.

3.3.13  network Remote ID(Net-RID)display provider;n—a logical entity that aggregates network Remote ID data from potentially multiple Net-RID service providers and provides the data to a display application (that is,an app or website);in practice,it is expected that many USSs may be both Net-RID display  providers  and  Net-RID  service  providers,but  stand- alone Net-RID display providers are possible.

3.3.14 network publishing,v—the act of transmitting data to an internet service or federation of services;clients,whether air traffic  control(ATC),public  safety  officials,or  possibly  the general public can access the data to obtain ID and tracking information for UAS for which such data has been published.

3.3.15 networked UAS,n—a UAS that during operations is in electronic communication with a Net-RID service provider (for  example,by means  of  internet Wi-Fi,⁴cellular,or satellite,or other communications medium such as short burst data satellite communications).

3.3.16 non-equipped UAS,n—in the context of Remote ID, a UAS that is neither a networked nor broadcast UAS  (for example,a radio controlled model aircraft)and cannot directly report its location or identity.

3.3.17 operator;n—the individual or organization who uses, causes to use,or authorizes to use an aircraft for the purpose of air  navigation,including  the  piloting  of  an  aircraft,with  or without the  right of legal  control (as owner,lessee,or otherwise). F3060

3.3.18 operator location,n—the geographic location of the remote pilot in command of a UAS.

3.3.19 position extrapolation,n—a capability of a Net-RID service provider to predict the location of a UAS based on a modeled 4-D trajectory derived from an intended UAS opera- tion plan.

3.3.20  registration,n—the  process  by  which  an  owner/ operator  (including  contact  information  and  other  PII)and aircraft(for  example,make,model)are  associated  with  an assigned,unique  identifier.

3.3.21  shall,must  versus  should  versus  may—use  of  the word "shall"implies that a procedure or statement is manda- tory  and  must  be  followed  to  comply  with  this  practice, "should"implies recommended,and  “may”implies  optional  at the discretion of the  supplier,manufacturer,or operator.

3.3.21.1  Discussion—Since "shall"and“must”statements are requirements,they include sufficient detail needed to define compliance  (for  example,threshold  values,test  methods, oversight,and  references  to  other  standards).“Should”state- ments also represent parameters that could be used in safety evaluations,and could lead to development of future require- ments.“May”statements  are provided  to  clarify  acceptability of a specific item or practice,and offer options for satisfying requirements.

3.3.22 static data,n—data that remains the same or does not change often over the duration of a flight(for example,Unique ID);this is in contrast to dynamic data that may change more frequently(such as longitude and latitude).

3.3.23  UAS  operation  plan,n—a  UAS  operation  plan  is developed  prior to  the  operation  and  should  indicate  the volume of airspace within which the operation is expected to occur,the times and locations ofthe key events associated with the  operation,including launch,recovery,and any other information deemed important (for example,segmentation of the operation trajectory by time).

UTM ConOps v1.0

3.3.24 UAS registration ID,n—an identification number or combination  of letters  and  numbers  assigned by  a  CAA  or authorized representative to a UAS;this is sometimes referred to  as  a  registration  number(which  may  or  may not  contain letters).

3.3.25  UAS  service  supplier(USS),n—USSs provide  UTM services to support the UAS community,to connect operators and other entities to enable information flow across the USSnetwork,and to promote shared situational awareness among UTM participants.

UTM ConOps v1.0

3.3.26 unique ID,n—a data element that can be traced to a unique UAS and its operator.

3.4 Acronyms and Abbreviations:

3.4.1 AES,n—advanced encryption standard

3.4.2 AGL,adj—above ground level

3.4.3 API,n—application programming interface

3.4.4 ARC,n—aviation rulemaking committee

3.4.5 BVLOS,adj—beyond visual line of sight

3.4.6  C2,n—command  and  control

3.4.7 CAA,n—Civil Aviation Authority

3.4.8 CONUS,n—contiguous United  States

3.4.9 DAR,n—DSS airspace representation

3.4.10 DSS,n—discovery and synchronization service

3.4.11 EIRP,n—effective isotropic radiated power

3.4.12  EMI,n—electromagnetic  interference

3.4.13 FAA,n—Federal Aviation Administration

14 Used throughout the specification,Wi-Fi is a registered trademark of Wi-Fi Alliance,10900-B Stonelake  Boulevard,Suite 126,Austin,TX 78759.

3.4.14 GCS,n—ground control  station

3.4.15 Hz—Hertz(cycles per  second)

3.4.16 inHg—inch of mercury

3.4.17 km—kilometers

3.4.18 kts—knots(nautical miles per hour)

3.4.19 LAANC—low altitude authorization and notification capability

3.4.20 LE—little endian (least significant byte first)

3.4.21 LSB—least significant bit

3.4.22 LTA—lighter than air (for example,balloon or blimp)

3.4.23 m—meters

3.4.24 m/s—meters per second

3.4.25 mb—millibars

3.4.26  mm—millimeters

3.4.27 MAC—media access control

3.4.28 MPH—miles per hour

3.4.29 MSB,n—most significant bit

3.4.30 Net-RID,n—network Remote ID

3.4.31 PHY,n—physical layer

3.4.32 PII,n—personally identifiable information

3.4.33 PPM—parts per million

3.4.34  Remote  ID,n—remote  identification

3.4.35 TLS,n—transport layer security

3.4.36  UA,n—unmanned  aircraft

3.4.37 UAS,n—unmanned aircraft system

3.4.38 USS,n—UAS service supplier

3.4.39 UTM,n—UAS traffic management

3.4.40  UUID,n—universally  unique  identifier  based  on RFC4122(128 bit)

3.4.41  VIP,n—very  important person

3.4.42 VLL,adj—very low level(airspace—generally below 150m(500  ft))

4.Remote ID and Network Interoperability Conceptual

Overview

4.1 This section provides a conceptual overview of Remote ID as defined in this specification,explains the scope of the specification,and  clarifies the  differences between broadcast and network Remote ID.This overview does not address all nuances  of  the  specification.The  intention  is  to  provide  a contextual  framework  to  understand  the  requirements  con- tained in this  specification.No requirements are provided in this  section.

4.2 This section also provides an overview of the general approach to interoperability between USSs for both Network Remote ID and other UTM-related services.

4.3 Scope of Standard and Remote ID Components:

4.3.1  Fig.1 identifies  the  actors  and  interfaces  between actors in the Remote ID environment.

4.3.2  The  scope  of this  specification  is  identified  by  the contents of the dotted purple box in the center of the diagram.

4.4 Broadcast Remote ID:

4.4.1 Broadcast Remote ID is depicted in the upper,central portion of Fig.1 in  blue.Equipment on  participating  UAS continuously transmit Remote ID data using one of the transmit protocols in this  specification(Bluetooth or Wi-Fi).It is possible that additional transmit protocols may be added in the future as warranted by available technology.The initial technologies were selected for compatibility with commonly car- ried hand-held devices that have their own receiver antenna. However,equipment to receive the broadcast data is not part  of the specification.Other implementations,such as receivers not integrated with hand-held devices,are possible.

4.4.2  Both  Bluetooth  and  Wi-Fi  continuously  broadcast messages to advertise the presence of the associated device. These advertisements normally allow other devices to discover  and establish connections with the associated device,but the advertisements themselves can carry a payload.These adver- tisements contain the broadcast Remote ID data.A hand-held device  does  not  need  to  establish  a  connection  to  receive Remote ID data,instead it need only receive and process the advertisements.

4.4.3  Broadcast  Remote  ID  can  be  used  anywhere,but  is necessary  in  areas  where network  coverage  is  unreliable, disrupted,or  not  available.

4.4.4 The specification also includes a range of options for authentication of broadcast data.Some of those options include digital  signatures  over  portions or  all  of  the  Remote  ID message  set.While  the  specification  does  not  specify  the encoding format associated with signatures,it does include a standard API that would be used by a receiver of the broadcast data(for example,an app on a smartphone)to contact a verifier with the signature data for a broadcast to determine message validity.This  is  described  in  more  detail  in Annex  A1, Broadcast Authentication Verifier Service.

Scope of Standard

Broadcast  UAS

Bluetooth  4.x,5.x

Wi-Fi

Verifier  API

Networked UAS

Direct from Drone or GCS

Users

Net-RID

Service Provider

(USS)

Intent-Based

Operation Reported

Preflight

Discovery of Relevant Service Providers

Obtain Data for Relevant UAS

Fixed Flying Fields,

other published

aeronautical data

Public,Law

Enforcement,

others

Net-RID Display  Provider 	(USS)

Display App

FIG.1 Remote ID Conceptual Overview

4.5 Network Remote ID:

4.5.1 Network  Remote  ID  can  be  used  when  both  UAS operations and end users of Remote ID display applications access  the  internet,typically  by  means  of  cellular  network. Cellular coverage tends to be higher in urban areas.

4.5.2 Network Remote ID is depicted in the lower,central portion of Fig.1 in orange and enclosed in a dashed line cloud. The nominal case supports Networked UAS(that is,UAS that remain in contact with a Remote ID Service Provider during flight either directly or through an intermediate device such as a ground control station),although the specification accommodates intermittent loss of network connectivity.Network Re- mote ID also includes provisions for participation in Remote ID through intent-based preflight reporting,also referred to as Intent-Based participation.This technique provides an alternative  for  UAS  that  are  non-equipped  (that  is,UAS  that  are neither broadcast capable nor equipped to communicate with a Remote  ID  Service  Provider  during  flight,such  as  many  radio-controlled model aircraft)as well as for UAS that may be equipped but are operating in an environment where participation in either broadcast or network Remote ID is infeasible due to the presence of electromagnetic interference (for example, an inspection inside an electrical tower)or radio signal blocking material (for example,areas under a bridge shielded  by thick concrete or inside a tunnel).Intent-based participants report  their  operations(for  example,aircraft  ID,location  in terms of a volume of airspace,operating times)in advance.The information  is  used  to  create  a  position  report  for  use  by Remote ID display applications where the uncertainty of the position  report  is  defined  by  the  airspace  volume  for  the operation.The  current  telemetry  of  the  aircraft  within  the volume is not known and cannot be displayed to a Remote ID  end user,but the display application can display the volume and provide the identity of the UAS.

4.5.3 For Network Remote ID,two USS roles are identified: Network Remote ID(Net-RID)Service Providers and Net-RID Display Providers.In practice,these roles can be fulfilled by a single  USS  and  potentially  one  that  also  provides  flight planning  and  deconfliction,LAANC,or  other  UTM  services, or  combinations  thereof.However,they  are  identified  sepa- rately to provide flexibility for industry participants to pursue their preferred business objectives and implementation scope. This  architecture  supports  one  or  more  Net-RID  Service Providers and one or more Net-RID Display Providers.

4.5.4 Net-RID Service Providers nominally remain in con- tact  with  UAS  during  flight  and  receive  information(for example,position  updates)used  to  fulfill  requests  from  Net- RID  Display  Providers.For  Network Remote  ID,some  re- quired data(for example,the UAS ID)may be retained by the Net-RID  Service provider after UAS authentication and not transmitted continuously from the UAS.As this specification does not specify the details of the UAS to Net-RID Service provider interface,implementations are generally valid as long as  complete  and  correct  Remote  ID  data  is  obtained  by Net-RID Service Providers at some point and made available to Net-RID Display Providers.

4.5.5 Net-RID Service Providers may also have the ability to  supply  extrapolated  position  information  for  UAS  that intermittently lose network connectivity.

4.5.6 Net-RID Display Providers  fulfill a broker role be- tween Remote ID Display Applications used by an end user and all Net-RID Service Providers that have flights in an area. When an end user display application requests Remote ID data  for an area,the Net-RID Display Provider servicing the display application determines what Net-RID Service Providers have operations in the area and then obtains appropriate Remote ID  data from each.The aggregated data is returned to the Remote ID  Display  Application.The  aggregated  data  includes  both current location and a window of near-real-time data for each flight.

4.5.7 Net-RID Display Providers ensure Remote ID Display Applications can only access and view data within a limited range  and  must  dispose  of  aggregated  data  obtained  from Net-RID  Service  Providers  within a  defined  time  period. Limiting the range helps implementations satisfy performance requirements in this specification by bounding the volume of data that must be gathered,processed,and displayed.Limiting the range (that is,only accessing required data)and disposing of such data when no longer needed helps protect privacy and sensitive data of consumers and operators.

4.5.8 For a UAS to be included in response to queries for Remote ID data,it must either be within the requested area at the  time  of  the  request  or  recently  therein(that  is,within  a small window  of time  such  as  a  minute).This  specification does not provide remote identification data for UAS that are projected to be within an area in the future.

4.5.9  Industry-standard  encryption  and  authentication  are required  from  the  UAS  or  the  operator  of  an  intent-based network  participant  to  the  Net-RID Service  and  from  the Net-RID Service Providers to Net-RID Display providers.

4.6 Remote ID Display Applications:

4.6.1 Receivers  and  Remote ID Display Applications  are shown on the right side of Fig.1. A typical implementation would be a smartphone or tablet with an internal receiver for Bluetooth and Wi-Fi,but other implementations are possible. The display applications ingest Broadcast Remote ID data or interact with a Net-RID Display Provider,or both,to acquire Network Remote ID data and present the information to end users.

4.6.2  A  typical  user  interface  might  be  map-based  with symbols for UAS in the area.However,the manner in which the  information  is  presented  is  beyond  the scope  of  this specification and other implementations are possible.

4.6.3 It is anticipated that Remote ID Display Applications that integrate Broadcast and Network Remote ID data will be produced by industry;however,this also is beyond the scope of the specification.

4.6.4 From a network Remote ID perspective,this specifi- cation levies performance requirements on Net-RID Display Providers in responding to requests from Remote ID Display Applications.

4.7 Representative Remote ID Scenario:

4.7.1  Fig.2 depicts  a  representative  Remote  ID  scenario. The text that follows describes the flow of information amongst the Remote ID components introduced above.

4.7.2  Three  UAS  are simultaneously  operating  in  close proximity(within 1 km)to each  other:one  is  broadcasting Remote ID data,one is networked,and one is a model aircraft with no broadcast or network capability.An interested observer wants to identify the three UAS.

4.7.2.1 The broadcast UAS transmits Remote ID data using one of the methods described in 5.4. The UAS is controlled locally by the Remote Pilot and has no interface with a USS.

4.7.2.2 The networked UAS is operated under USS1.This USS  acts  as  a  Net-RID Service  Provider  and  a  Net-RID  Display Provider.

4.7.2.3  The  Remote  Pilot of  the  model aircraft  uses a smartphone  app  to  report  the  location  and  time  of  the operation,and  provides  the  ID  for  the  model  aircraft.The smartphone app is the user interface that connects the user to a second Net-RID  Service  Provider,USS2.

4.7.3 The interested observer accesses a Remote ID Display Application(RID App)that uses USS1 as its Net-RID Display Provider.This display application shows UAS locations and a near-real-time trail of position reports on a map,and associated identification information when a particular UA is selected.

4.7.4 When the interested  observer opens the Remote ID app  on  a  smartphone  and  centers  the  map  on  the  current location,Remote ID data is acquired as follows:

4.7.4.1 The broadcast UAS is transmitting its Remote ID  advertisements continuously.The smartphone uses its internal radios to listen for the advertisements from the UAS,extract the Remote ID data,and show the location of the UA on the map.As new position updates are received,the prior position reports become part of a near-real-time trail representing where the UA most recently flew.

4.7.4.2 Simultaneously,the Display App makes a request to its Net-RID Display Provider,USS1.USS1's role as a Net-RID Display Provider is to aggregate Remote ID data for all flights in  the  area  managed  by  Net-RID Service  Providers.USS1 knows that it is a Net-RID Service Provider and has flights in the area.

RC Model Aircraft

(Intent-Based  Network

Participant)

Control Link:

Control Link (1-way)

Reported  Operation

Operator

USS2

Net-RID Service

Provider

---Remote  ID  Requests

Display Provider

FIG.2 Representative Remote ID Scenario

USS

Interface Operator

Remote ID Display

Application

Net-RID Service

Provider  &

Broadcast UAS  Networked UAS

Control Link (2-way)

..UA Control Links

—  USS Interface

Interested Observer

Requests- Data

(2-way):

RID APP

Operator

USS1

&Data

FIG.2 Representative Remote ID Scenario

4.7.4.3 USS1 additionally discovers USS2,which has no real-time-managed flights in the area,but has an operation reported for the model aircraft(that is,the Intent-Based Network Participant).The Remote Pilot of the radio-controlled model aircraft reports the operation to USS2 prior to the flight and no dynamic position updates are provided.USS2 provides the information for this operation back to USS1.

4.7.4.4 USS1 adds the Remote ID data for the networked UAS that it is managing(fulfilling its role as a Net-RID Service Provider)and provides the aggregated data back to the Display Application(fulfilling its role as a Net-RID Display Provider).

4.7.4.5 The display app adds the network data to the map that  is  already  showing  the  broadcast  information.The  Net- worked UAS is shown with up to a 60 s trail of position reports (referred to as near-real-time data).The Intent-Based Network Participant is shown as a polygon.

4.7.4.6 As long as the interested observer continues to view the Remote ID display app for the area,the app continues to communicate with USS1 as its Net-RID Display Provider to obtain position updates.Since the information for the Intent- Based  network  participant  does  not  update,no  additional updates  are  provided  for  it  from  USS2.USS1  continues  to provide position updates for the Networked UAS in its role as the Net-RID Service Provider.

4.7.4.7 The interested observer selects the UA symbols for the Broadcast UAS and the Networked UAS on the map and views  the  corresponding  ID  information.The  interested  ob- server then selects the polygon for the Intent-Based Network Participant and sees the operation schedule and the ID of the aircraft

4.7.4.8 The interested observer  closes  the  app.After  a period of time,USS1 discards the information for the Intent- Based Network Participant because it is managed by a different USS(USS2).

4.8  USS Interoperability:

4.8.1 This  specification  assumes that UTM  services in a given location  are  provided by a set of one or more UAS Service  Suppliers(USSs). USSs must be  interoperable  in this environment,sharing data as necessary to accomplish  the objective of a particular service such as Network Remote ID or flight plan exchange for strategic deconfliction.

4.8.2 The interoperability paradigm defined in this specifi- cation is intended to support both Network Remote ID and other services that may be included in subsequent UTM-related ASTM standards.The requirements and application program- ming interfaces(APIs)associated with  the interoperability paradigm are included in this document because it is the first UTM-related  ASTM standard.Subsequent  UTM-related ASTM  standards  may  introduce  additional service-specific interoperability requirements and API functions.Some of the interoperability requirements and APIs may move to a different standard in the future.

4.8.3 The interoperability paradigm consists of two parts:

4.8.3.1 A standardized discovery mechanism,referred to as the Discovery and Synchronization Service(DSS),the primary functions  of  which  are  to  identify  USSs  with  which  data exchange  is  required,and  to  verify  that  a  USS  considered relevant  information  from  other  USSs  when  necessary(for example,when  planning  a  new  operation);and,

4.8.3.2  Service-specific  data  exchange  protocols  used  to obtain the details of relevant information discovered by means of the DSS from the owning USS.

Discovery &Synchronization Service

DSS

Airspace

Representation

2)Discover

1 )Make Discoverable

·1-time  request  or subscriptions

USS with

Data other

USSs need

4

3

Subscription Notifications

)Get  Details  Other USSs

that  need data

Data Exchange Protocol

FIG.3 USS Interoperability Overview

4.8.4  Fig.3  illustrates  the  interactions  involved  in  this paradigm in a service-independent manner.(The instantiation of this paradigm for Remote ID is detailed later.)

4.8.5 DSS-related interactions are shown at the top in the blue-shaded area;data exchange protocols between USSs are represented by the green-shaded area.

4.8.6  For  availability  purposes,the  DSS  is  a  redundant service as indicated in Fig.3 .Instances of the DSS in a region synchronize  with  each  other  in  a  standardized  manner(described later in this specification).A region is the geographic scope supported by a set of DSS instances.

4.8.7 Only approved USSs will be given access to the DSS. (The specifics of an approval process are beyond the scope of this specification.)

4.8.8 An instance of discoverable information is referred to as  an  entity.There  are  different  types  of  entities,such as operations,constraints,or,relevant to this specification,an area where network remote identification services are being provided.The concept is extendable to future UTM services where other  types  of  entities  may  need  to  be  discoverable.A  key characteristic of entities is that they have an associated 4-D volume(that  is,a  volume  defined  in  x,y,and  z  plus  time limits).

4.8.9 The DSS encapsulates an airspace model into which entities  are  mapped.The  implementation  details  of  this  air- space  representation  are  hidden  from  DSS  clients;however, conceptually,the airspace model can be thought of as a grid and mapping an entity into the airspace model is determining what grid cells the entity intersects.The complete details of the entities and any associated Personally Identifiable Information (PII)are not stored in the DSS but instead are retained by the owning  USS;only  limited  information  such  as  the  type  of entity,its location (in terms of what cells of the airspace model it intersects),the current opaque version number(OVN)of the entity(OVNs are updated whenever the entity is modified),and how to contact the owner of the entity are stored in the DSS.

4.8.10  Given  that  context,the  primary  interactions  (numbered in Fig.3 )are:

4.8.10.1 Make Discoverable — A  USS with  an entity about which other USSs need to know(for example,an operation,a constraint,an  area  where  Remote  ID  services  are  provided) makes it discoverable by writing the limited entity summary information (information type,identifier,location,owner)to the DSS.

4.8.10.2 Discover—Other USSs that are interested in entities  of  some  type  query  the  DSS  using  a  4-D  volume  to characterize the area of interest.The DSS maps the query onto the airspace representation and finds intersecting grid cells with entities  of  the  desired  type  (if  any).(Because entities  are mapped into grid cells and the DSS does not retain the precise extents,the DSS will occasionally return an entity that does not intersect the precise area of interest;however,it will never omit an  entity  that  intersects  the  area  of  interest.)The  DSS  then returns to the requesting USS a list of the discovered entities and their owners.This can be a one-time query(often described as a pull of the information)or the requesting USS can also establish  a  subscription  to  be  notified  of new  or  modified entities in the area of interest (discussed further below).

4.8.10.3 Get Details—Given the list of discovered entities, the requesting USS switches to the applicable Data Exchange Protocol to contact the owning USS and obtain the complete details.Data Exchange Protocols are service-specific.

4.8.10.4 Subscription Notifications—If the requesting  USS established  a  subscription  in  the  DSS  (for  a  4-D  area  of interest),when another USS writes a new entity to the DSS that intersects the subscription,the DSS informs the writing USS of the subscription and the writing USS contacts the subscribing USS to provide the details.(This is often described as a push of the information.)

4.8.11 While not needed for Remote ID,OVNs come into play on interaction if the entity written to the DSS is of a type that requires  deconfliction  with  other  entities(for  example,a new UAS  operation requires deconfliction;a  constraint does not).When  writing  the  new  operation  to  the  DSS,the  USS must provide the OVNs for all other operations and constraints in the area of the new operation.For applicable entity types, OVNs are part of the detailed information obtained from other USSs  in  step  3.If the  DSS  determines  the  set  of  OVNs  is complete and current,it allows the new operation to be written; if not,the  DSS  informs  the  writing  USS  what  OVNs  are missing or obsolete.

4.8.12 Although complete details for entities are not stored in the DSS,it  serves  as  the  single  source  of truth  for  what entities  exist  in  the  airspace  and  provides  the  mechanism necessary  to  ensure  that  USSs  attempting  to  create  a  new operation  have  considered  the  current  version  of  all  other relevant entities in the airspace.

4.8.13 Unless noted otherwise,references to the DSS throughout this specification refer to the set of DSS instances supporting the region in which a related activity is occurring (for  example,creating  entity  summaries,discovering  entities).

4.8.14 This overview omits many details of the DSS and data  exchange  protocols.The  Remote  ID-specific  interoperability requirements,complete APIs,and additional details are provided in Annex A2.

5.Performance Requirements

5.1 Remote ID is comprised of a set of standardized data, messages,transport  mechanisms  for  communicating  the messages,and  performance  requirements  governing  certain attributes of an implementation such as message periodicity. For  Broadcast  Remote  ID,the  message  format  is  the  same regardless  of  the  transport  mechanism.These  messages  are coded  as  a  “block  message”implementation  of  the Data Dictionary  to  optimize  for  the  transport  mechanism  size constraints and to minimize potential broadcast interference. For  Network  Remote  ID,the  message  format  is  a  network adaption of the Data Dictionary using common internet proto- cols.For broadcast messages,each message has a message type that  is  identified  in  the  message  header.The  message  type defines  the  message format  and  is  classified  as  static  or dynamic,which also sets the requirements for the minimal rate at which each message type shall be transmitted.The common name for this broadcast messaging protocol is “Open Drone ID."

5.2 Conventions used in this section:

5.2.1 Requirement IDs are shown below.The prefix to each ID identifies groupings:

5.2.1.1  BURxxxX-Broadcast  Update  Rate

5.2.1.2  BMGxxxx  -Broadcast  Messages

5.2.1.3  BB4xxxx  -Broadcast  Bluetooth  4

5.2.1.4  BB5xxxx  -Broadcast  Bluetooth  5

5.2.1.5  BWFxxxx  -Broadcast  Wi-Fi

5.2.1.6  NETxxxx  -Network

5.2.1.7 DSSxxxX -Discovery and Synchronization Service (Annex A2)

5.2.2 Constant values representing a required time,distance, etc.are consolidated into Annex A3.These values are  referenced within the requirements text in this section using square brackets around the constant name.Constants  pertaining to broadcast Remote ID are prefixed with “Bc”and constants pertaining to network Remote ID are prefixed with "Net"as shown in the following examples:

5.2.2.1  [BcMinUasLocRefreshRate]

5.2.2.2  [NetMinUasLocRefreshRate]

5.2.3 In some cases,notes to clarify the intent of a requirement are provided.These notes are numbered and prefaced with “Note:.”They are for clarification purposes only and do not contain requirements.

5.3 Common Data Dictionary:

5.3.1  Table 1  defines the required and optional data fields for Remote ID,including minimum characteristics that must be supported  by  both  network  and  broadcast  implementations. Since  broadcast  Remote  ID  uses  size-limited  messages,for some data fields it is necessary to use encoding methods that adjust  the resolution  or  aggregate  ranges  of values,whereas these size-limiting techniques are not necessary for network Remote  ID.The  required  minimum  characteristics  provided below  ensure  a  prescribed  degree  of  consistency  between broadcast and network Remote ID to facilitate integration in a Remote ID display application.The specific representations for broadcast  and  network  Remote  ID  are  provided  in  their respective requirements sections.

5.3.2 An asterisk(*)adjacent to a data  field name denotes an optional field.Optional fields and certain field options may be required in some jurisdictions.

TABLE1 Common Data Dictionary

| Data Field | Description/Rationale |
| --- | --- |
| UAS ID | Consists of four options: 1.Serial Number:This is expressed in the CTA-2063-A Serial Number format. 2.Registration ID:If a CAA provides a method of registering UAS,this number is provided by the CAA or its authorized representative. Format:<ICAO Nationality Mark⁴>.<CAA Assigned ID>,ASCII encoded,only uppercase letters(A-Z),dot (.),and digits(0-9)are allowed.Example(US):N.123456 3.UTM(UUID):A UTM-provided universally unique ID traceable to a non-obfuscated ID where this UTM UUID acts as a “session id” to protect exposure of operationally sensitive information. 4.Specific Session ID:A unique 20 byte ID intended to identify a specifi fight (session)while providing a greater level of privacy to the operator.The first byte is the registered unique Specific Session ID Type maintained by a registrar(see Annex A5), and the remaining 19 bytes is the Session ID in accordance with the Specific Session ID Type specification. Initial scheme registry entries shall include: 0-reserved; 1-Internet Engineering Task Force(IETF)Drone Remote Identification Protocol(DRIP)entity ID; 2-IEEE 1609.2-2016 HashedlD8. |
| UA Type | The UA Type can help infer performance,speed,and duration of flights,for example,a "fixed wing"can generally fly in a forward direction only (as compared to a multi-rotor).This can also help diferentiate aircraft types without sharing operationally sensitive information like the make and model of a particular aircraft.Make and model are anticipated to become available during the Registration ID lookup process.UAS Type is also useful for core lating visual observation with data received.The types were formulated based on unique flight characteristics.The possible values are in Table 2. |
| UA Classification*C | UA Classification-Allows a region to classify UAS in a regional specific manner. The format may differ from region to region. |
| UA Classification Type | Specifies the Region Number (which implies the format of UAS Classification).See Table 2. |
| Timestamp | The time of applicability of position information.This may be the time coming from the source such as a GPS,or the time when the system computes the values using an algorithm such as an Extended Kalman Filter(EKF).Timestamps must be expressed with a minimum resolution of one tenth of a second and relative to UTC time. Special Values:Invalid,No Value,or Unknown:0xFFFF. |
| Timestamp Accuracy | Declaration of timestamp accuracy,which is the largest iference between Timestamp and true time of applicability for any of the fol- lowing fields:Latitude,Longitude,Geodetic Altitude,Pressure Altitude,and Height to determine time of applicability of the location data provided.Expressed in 1/10ths of seconds.The accuracy reflects the 95%uncertainty bound value for the timestamp. |
| Operational Status*C | Operational Status indicates whether the associated UA is on the ground,airborne,in an emergency situation,or the Remote ID system has failed.The emergency status takes precedence over the other status modes.This status can be used for filtering purposes. (See Table 2.) |
| Operation Description*C | This optional,free-text field enables the operator to describe the purpose of a flight,if so desired. |
| Operator ID*C | This optional field provides a CAA-issued registration/license ID for the remote pilot or operator. Format:ASCII Text.If numeric values exist,they shall be expressed as a string of ASCII characters. |
| Latitude | Current latitude (within horizontal accuracy limits)of the UA.This is necessary to display UAS location.Minimum resolution:7 decimal digits(~11mm). Special Values:Invalid,No Value,or Unknown:0 deg(both Lat/Lon) |
| Longitude | Current longitude (within horizontal accuracy limits)of the UA.This is necessary to display UAS location.Minimum resolution:7 decimal digits(~11 mm). Special Values:Invalid,No Value,or Unknown:O deg(both Lat/Lon) |
| Geodetic Altitude | The aircraft distance above or below the ellipsoid as measured along a line that passes through the aircraft and is normal to the surface of the WGS-84 ellipsoid.This value is provided in meters and must have a minimum resolution of 1m. Special Values:Invalid,No Value,or Unknown:-1000m |
| Pressure Altitude*C | The uncorrected barometric pressure altitude (based on reference standard 29.92 in Hg,1013.25 mb)provides a reference for algorithms that utilize “altitude deltas”between aircraft.This value is provided in meters and must have a minimum resolution of 1m. Special Values:Invalid,No Value,or Unknown:-1000 m |
| Height*C | Expressed as either height above takeoff location or height above ground level (AGL)for a UA's current location.This value is provided in meters and must have a minimum resolution of 1 m. Special Values:Invalid,No Value,or Unknown:-1000 m |
| Height Type*C | Height above takeoff location or height above ground level. |
| Geodetic Vertical Accuracy | Provides quality/containment on geodetic alitude.This is based on ADS-B Geodetic Vertical Accuracy (GVA)(plus the three extra increments).(See Table 2.) |
| Horizontal Accuracy | Provides quality/containment on horizontal position.This is based on ADS-B NACp(plus the one extra increment).(See Table 2.) |
| Data Field | Description/Rationale |
| Speed Accuracy | Provides quality/containment on horizontal ground speed.(See Table 2.) |
| Track Direction | Direction of flight expressed as a "True North-based"ground track angle.This value is provided in clockwise degrees with a minimum resolution of 1 degree.If aircraft is not moving horizontally,use the “Unknown”value. Special Values:Invalid,No Value,or Unknown:361 deg |
| Speed | Ground speed of flight.This value is provided in meters per second with a minimum resolution of 0.25 m/s. Special Values:Invalid,No Value,or Unknown:255 m/s,if speed is ≥254.25 m/s:254.25 m/s |
| Vertical Speed | Vertical speed upward relative to the WGS-84 datum,meters per second. Special Values:Invalid,No Value,or Unknown:63m/s,if speed is≥62 m/s:62 m/s,if speed is≤-62 m/s:-62 m/s |
| Auth Data*C | Additional Authentication Data See Table 2 for specific Authentication Types |
| Operator Latitude*C | Special Values:Invalid,No Value,or Unknown:0 deg(both Lat/Lon) Provides the location associated with the Remote Pilot. |
| Operator Longitude*C | Provides the location associated with the Remote Pilot. Special Values:Invalid,No Value,or Unknown:0 deg(both Lat/Lon) |
| Operator Altitude*C | Provides the Operator Altitude based on WGS-84 height above ellipsoid(HAE)(See Geodetic Altitude). This value is provided in meters and must have a minimum resolution of 1 m. Special Values:Invalid,No Value,or Unknown:-1000 m |
| Operator Location Type*C | Takeoff location,fixed location,or dynamic location representing the Operator Location |
| Operating Area Radius*C | Farthest horizontal distance from the reported location at which any UA in a group may be located (meters).Also allows defining the area where an Intent-Based Network Participant operation is taking place.Default:0 |
| Operating Area Polygon*C | A list of latitude/longitude pairs defining the area where a group or Intent-Based Network Participant operation is taking place.(This field is only applicable to Network Remote ID.) |
| Operating Area Type*C | Cylinder or Polygon.(This field is only applicable to Network Remote ID.) |
| Operating Area Count*C | Allows for operating a single UA,group,formation,or swarm:Quantity in Group.Default 1 |
| Operating Area FloorC | Minimum altitude(WGS-84 HAE)for a group or an Intent-Based Network Participant. Special Values:Invalid,No Value,or Unknown:-1000 m |
| Operating Area Ceiling*C | Maximum altitude(WGS-84 HAE)for a group or an Intent-Based Network Participant. Special Values:Invalid,No Value,or Unknown:-1000 m |
| Operation Area Start*C | The date and time at which a group or an Intent-Based Network Paricipant operation starts.(This field is only applicable to Network Remote ID.) |
| Operation Area End*C | The date and time at which a group or an Intent-Based Network Participant operation ends.(This field is only applicable to Network Remote ID ) |

A ICAO Nationality Marks,https://www.icao.int/safety/airnavigation/Pages/nationality.aspx.

B “resolution”in this specification is used to indicate the preciseness possible of the expressed value,but not the accuracy.For example,although  Lat/Lon  must  be expressed to 7 decimal digits of resolution,the accuracy may be far less(as indicated in the Horizontal Accuracy field).

C An asterisk(")adjacent to a data field name denotes an optional field.Optional fields and certain field options may be required in some jurisdictions.

5.4 Broadcast:

5.4.1 This section describes requirements for the RF broadcast  of Remote  ID  messages  from  a  participating  UA.Four broadcast transport mechanisms are supported by this specification:

5.4.1.1  Bluetooth  Legacy(4.x  compatible)

5.4.1.2  Bluetooth  5.x  Long  Range(must  be  transmitted concurrently with Legacy mode)

5.4.1.3 Wi-Fi Neighbor Awareness Network(NAN)

5.4.1.4 Wi-Fi Beacon(vendor-specific information element in the SSID beacon frame)

5.4.2  The  four  transport  mechanisms  share  common  requirements for update rates and message definition.

5.4.3  Output Power  and Pattern—For  output power  and pattern,the  requirement(BPW0010)seeks  to  provide  a  sufficiently  high  power  transmission  that  generally  emits  in  an omnidirectional  pattern  using  commonly  available  components.This can be accomplished using either of the following two options:

(a)The average Effective Isotropic Radiated Power(EIRP) around the horizontal plane of the antenna system shall be at least [BcMinAvgEIRP]and the Peak to Average gain around the horizontal plane of the antenna system shall be no more than [BcMaxPeakToAvg].The average EIRP is calculated by adding the conducted power into the antenna  system to the average gain of the antenna system in the horizontal plane;or

(b)The minimum EIRP around the entire horizontal plane of the  antenna  system  shall  be  at  least [BcMinEIRP].The minimum EIRP is calculated by adding the conducted power into the antenna system to the minimum gain of the antenna system in the horizontal plane.

These options are intended to allow for use of manufacturer data sheets to demonstrate compliance.The Horizontal Plane is defined as a plane of the transmission pattern that approximately corresponds to the horizontal plane during the most common average orientation of the vehicle when flying.The UAS should be mechanically designed in a way to minimize radio pattern distortion of Remote ID.

5.4.4 Update Rates:

5.4.4.1 For broadcast messages,dynamic messages  (as  indicated in the block message section)shall (BUR0010)be sent at least  every  [BcMinUasLocRefreshRate]second(s).Static messages  (as  indicated  in  the block message  section)shall (BUR0020)be sent at least every  [BcMinStaticRefreshRate]second(s)and  the  maximum  potential  time elapsed since the time of applicability of the dynamic fields in the  Location/Vector  Message  shall(BUR0030)be  no  older than [BcMaxDataAge].If a multi-sector antenna configuration is used,then these update requirements shall be applied to each sector.Should channel saturation block or interfere with transmission(as may occur due to “listen before talk”interfer- ence handling technique),the  system  shall(BUR0040)make  a "best effort"to transmit when the saturation level allows.

5.4.4.2 Counters—After each frame of the  same message type is updated,the Message Counter (for that message type) shall(BUR0050)be incremented and reset to 0 after OxFF is reached.Therefore,each message type(defined in Table 3)has a  separate  counter.For  Message  Packs(message  type  OxF), only 1 counter is maintained and updated each time any data  within the message pack is updated.If the data being transmit- ted  has  not  changed,incrementing  the  Message  Counter  is optional.If a  message  is  multi-paged(such  as  a  multi-page authentication message),then the Message Counter  shall (BUR0060)be the same for each page in the page set.

5.4.5 Block Messages:

5.4.5.1  The“Block”messages  are  designed  to  be  packed into  lightweight  direct  broadcast  packets  within  Wi-Fi  or Bluetooth  "Beacon  Advertisements."The  message  types  are identified in Table 3. Subsequent subsections further describe each message type.

5.4.5.2 Each message shall(BMG0010)be 25 bytes in length (padded with nulls as needed).

5.4.5.3 Each message shall(BMG0020)begin with a  1 byte header followed by 24 bytes of data,which shall be encoded as described in the “Message Details”table that corresponds with  each Message Type described below.Non-magnitude values,  strings,or IDs that may be or may not be numerical(such as  the  Unique ID)shall(BMG0030)be expressed in Network  Byte Order which reads in a left to right,most significant byte  (MSB)to least significant byte (LSB)order.Magnitude values  expressed as 16 or 32 bit integers(such as Latitude,Longitude,  Altitude,etc.)shall(BMG0040)be  expressed as  “little endian” (marked as “LE”in the "Message Details”tables below),  where the LSB is on the left and the MSB is on the right.If not  invoking an optional message,it is not necessary to send that message.Optional fields within messages being sent(see Table 1)shall(BMG0050)be filled in as stated in the corresponding block message format  and  if  opting out,or  the value  is unknown,shall be filled with nulls(Os)for string values or 0 for numeric unless an alternate default/unknown value representation is stated in Table 1. This allows the block message to stay properly aligned with the field  definitions.All  ASCII Strings  shall(BMG0060)be filled with nulls in the unused  portion of the field.In the data structures below,some fields are  enumerated values. Table 2 shall(BMG0065)be used to encode to those enumerations.

TABLE 2 Enumerated Field Definitions

| Field Name | Details | Notes |  |
| --- | --- | --- | --- |
| UA Type | 0:None/Not Declared 1:Aeroplane 2:Helicopter (or Multirotor) 3:Gyroplane 4:Hybrid Lift(Fixed wing aircraft that can take 5:Ornithopter 6:Glider 7:Kite 8:Free Balloon 9:Captive Balloon 10:Airship(such as a blimp) 11:Free Fall/Parachute(un powered) 12:Rocket 13:Tethered Powered Aircraft 14:Ground Obstacle 15:Other | off vertically) | Up to 16 Types These values were derived from the official ICAO UA Type list.Additional types were added if they had unique flight characteristcs. |
| Operational Status | 0:Undeclared 1:Ground 2:Airborne 3:Emergency 4:Remote ID System Failure 5-15:Reserved | Up to 16 Statuses |  |
| Horizontal Accuracy | 0:≥18.52 km(10 NM)or Unknown 1:<18.52 km(10 NM) 2:<7.408 km(4 NM) 3:<3.704km(2 NM) 4:<1852 m(1 NM) 5:<926m(0.5 NM) 6:<555.6m(0.3 NM) 7:<185.2m(0.1 NM) 8:<92.6m(0.05 NM) 9:<30 m 10:<10 m 11:<3m 12:<1 m 13-15:Reserved | This is the NACp enumeration from ADS-B.Value 12 was added for a more complete range for UAs.95%accuracy bound(estimated position uncertainty). |  |
| Vertical Accuracy | 0:≥150 m or Unknown 1:<150 m 2:<45m 3:<25m 4:<10 m 5:<3m 6:<1 m 7-15:Reserved | This is the GVA enumeration from ADS-B.Values 4-6 were added for UAs.95%accuracy bound. |  |
| Speed Accuracy | 0:≥10 m/s or Unknown 1:<10 m/s 2:<3m/s 3:<1 m/s 4:<0.3 m/s 5-15:Reserved | This is the same enumeration scale and values from ADS-B NACv.95% accuracy bound. |  |
| Operator Location Source Type | 0:Takeoff 1:Dynamic 2:Fixed |  |  |

TABLE  2  Continued

Field Name Details Notes

0:Undeclared 1:European Union UA Classifications Classification Type 2-7:Reserved For Classification Type 1(EU)Categories: 0:Undefined 1:Open 2:Specific 3:Certified 4-15:Reserved Classes: 0:Undefined 1:Class 0 2:Class 1 3:Class 2 4:Class 3 5:Class 4 6:Class 5 7:Class 6 8-15:Reserved Authentication Type 0:None 1:UAS ID Signature 2:Operator ID Signature 3:Message Set Signature 4:Authentication Provided by Network Remote ID 5:Specific Authentication Method 6-9:Reserved for Spec

A-F:Available for Private Use

TABLE 3 Open Drone ID Block Message Summary

| Msg Type | Message Name | Purpose |
| --- | --- | --- |
| 0x0 | Basic ID Message | Provides ID for UA,characterizes the type of ID,and identifies the type of UA |
| 0x1 | LocationVector Message | Provides location,altitude,direction,and speed of UA |
| 0x2 | Authentication Message | Provides authentication data for the UA |
| 0x3 0x4 | Self-ID Message^ System MessageA | Message that can be used by Operators to identify themselves and the purpose of an operation Includes Remote Pilot location and mutiple aircraft information(group)if applicable,and additional system information |
| 0x5 | Operator IDA | Provides Operator ID |
| OxF | Message Pack⁴ | A payload mechanism for combining the messages above into a single message pack.Used with Bluetooth Extended Advertising,Wi-Fi Neighbor Awareness Network,and Wi-Fi Beacon |

A Optional unless required by location jurisdiction.

MESSAGE HEADER (Table 4)

5.4.5.4 The message header includes the Message Type and Protocol Version  and  shall(BMG0070)be  sent  in  each  message.

TABLE 4 Message Header Details

|  | Header(1 | byte) | Message(24 bytes) |  |
| --- | --- | --- | --- | --- |
| Message Type (4 bits) Bits [7..4] |  | Protocol Version (4 bits) Bits [3..0] | Message Fields based on Message | Type |
| 0x0-0xF |  | 0x2 (0xF reserved for private use) | <Message Data > |  |

BASIC ID MESSAGE (Table 5)

5.4.5.5 Basic ID Message Type:Ox0,Static Periodicity, Mandatory

5.4.5.6  The  BasicID  message  includes  the  ID  Type,UA Type,and  the  Unique  ID.This  Unique  ID shall(BMG0080) default  to  the  Manufacturer  Serial  number.Once  the  UA  is provisioned,the UAS ID shall be(BMG0090)one of the following:

(1)Manufacturer  Serial  Number  expressed  in  the  ANSI/ CTA-2063-A Serial Number format.

(2)A Civil Aviation Authority(CAA)issued Registration ID for the UA formatted as described in Table 1.

(3)A UTM Assigned ID if operating within a UTM system (128-bit  UUID)binary  encoded,Network  Byte  Order.

(4)Specific Session ID according to the registered Session ID Type (see Annex A5).The first byte of the Specific Session ID must be the  Specific  Session  ID  Type(1-255)where 0 is reserved,1-224 is the  registered  type,and 225-255 are available for private experimental  use  only.If  using this method,the  Specific  Session  ID  Type  must  be  registered through the registrar(see Annex A5).Using the experimental range is not considered “compliant”to this  specification,but may be used for testing and experimentation.

NoTE 1—Multiple ID Types may be sent by sending multiple Basic ID messages.

NoTE 2—When using Specific Session ID Type,implementations may wish to use random rather than static MAC addresses(see Tables 14,15, 17 PDU Hdr).

TABLE 5 Basic ID Message Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | ID Type, UA Type | Bits 7..0 [0000][0000] ID Type:Bits [7..4] 0:None 1:Serial Number(ANSI/CTA-2063-A) 2:CAA Assigned Registration ID 3:UTM Assigned UUID 4:Specific Session ID UA Type:Bits [3..0] helicopter,fixed wing,hybrid lift,etc.(See Table 2 for more details.) | Up to 16 ID types Up to 16 UA types |  |
| 2 22 | 20 | UAS ID | UAS ID within the format of ID Type(padded with nulls) | Max.20 Bytes | N.123456 |
|  | 3 | Reserved |  |  |  |

LOCATION/VECTOR  MESSAGE(Table 6)

5.4.5.7  Location/Vector  Message  Type:Ox1,Dynamic Periodicity,Mandatory

5.4.5.8 The Location/Vector message provides the location, altitude,direction,and speed of the UA.Several of the fields require special encodings to better pack the data and to allow for more precise values.If indicated,the transmitted data shall (BMG0100)be encoded according to Table 7. Additionally, any fields that require a flag bit to be set shall(BMG0110)be set according to Table 7 as well.

TABLE6 Location/Vector Message Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Status,Flags | Bits [7..0][0000][0000] Operational Status:Bits [7..4] Flags Reserved:Bit [3] Height Type:Bit [2] 0:Above Takeoff,1:AGL E/W Direction Segment:Bit [1] 0:<180,1:≥180 Speed Multiplier:Bit [0] 0:×0.25,1:x0.75 | 0..15 statuses (See Table 2) Speed Multiplier enables speeds 254.25 m/s.Only use 1 when speed 63.75m/s and add 63.75. | up to exceeds |
| 2 | 1 | Track Direction | Direction expressed as the route course measured clockwise from true north.Encode as 0-179.If E/W Direction bit is set,then 180 should be added to the value. | 0-359 Unsigned Int (UInt) | 10 with E/W bit set=190 deg. |
| 3 | 1 | Speed | Ground Speed in m/s encoded as specified in Table 7 | Up to 254.25 m/s Ulnt | 20(enc)=5m/s |
| 4 | 1 | Vertical Speed | Vertical Speed m/s(+up,-down)Multiplier=0.5 | Up to±62 m/s(12.2k ft/min) | 15(enc)=7.5 m/s |
| 5 | 4 | Latitude | Latitude of UA deg*10^7 | Int signed (LE) | -11989298 |
| 9 | 4 | Longitude | Longitude of UA deg*10^7 | Int signed (LE) (11 mm precision) | 48123987 |
| 13 | 2 | Pressure Altitude | Pressure Altitude(Ref 29.92 inHg,1013.24 mb) (Altitude+1000 m)/0.5(LE) | -1000-31767m(107503 ft) 16 bit Ulnt (LE) | 2021(enc)= 10.5 m |
| 15 | 2 | Geodetic Altitude | WGS-84 HAE (Altitude+1000 m)/0.5 | -1000-31767m(107503 ft) 16 bit UINT (LE) | 2021(enc)= 10.5m |
| 17 | 2 | Height | Height above takeoff location or Height above ground(indicate with Height Type Bit) (Altitude+1000 m)/0.5 | -1000-31767m(107503 ft) 16 bit Ulnt (LE) | 2021(enc)= 10.5 m |
| 19 | 1 | Vertical/ Horizontal Accuracy | Bits [7..][0000][0000] Vertical (Geodetic):Bits [7..4] Horizontal:Bits [3..0] Vertical:See Verical Accuracy Enumeration Horizontal:See Horizontal Accuracy Enumeration | See Table 2 |  |
| 20 |  | Baro Altitude Accuracy/Speed Accuracy | Bits [7..0][0000][0000] Baro Altitude:Bits [7..4] See Vertical Accuracy Enumeration Speed:Bits [3..0] Based on Extended ADS-B NACv | Baro:see Table 2 Speed:see Table 2 |  |
| 21 | 2 | Timestamp | Time of applicability expressed in 1/10ths of seconds since the last hour relative to UTC time | 0-36000:16 Bit UInt (LE) | 3611=6 mins, 1.1 s after the hour |
| 23 | 1 | Reserved/ Timestamp Accuracy | Bits [7..0][0000][0000] Reserved:Bits [7..4] Timestamp accuracy:Bits [3.0] (*0.1 s stepping resolution) | Timestamp accuracy: 0.1 s-1.5 s 0=unknown |  |
| 24 | 1 | Reserved |  |  |  |

TABLE 7 Encoding Table

| Field Type | To Encode (sender) | To Decode (receiver) |
| --- | --- | --- |
| Direction | If Value<180 EncodedValue =Value Set Direction Segment bt to 0 else EnclodedValue=Value-180 Set Direction Segment bit to 1 | if Direction Segment bit=0 Value=EncodedValue else Value =EncodedValue +180 |
| Speed(UInt8) | If Value-≤255*0.25 EncodedValue=Value/0.254 Set Multiplier Flag to 0 else if Value>255*0.25 and Value <254.25 EncodedValue=(Value-(255*0.25))/0.75A Set Multiplier Flag to 1 else(Value ≥254.25 m/s) EncodedValue=254A Set Multiplier Flag to 1 | If Multiplier Flag=0 Value=EncodedValue*0.25 else if Multiplier Flag=1 Value=(EncodedValue*0.75)+(255*0.25)B.C Encoding Rationale:This allows for a higher speed precision of 0.25 m/s for lower speeds(0.5 kts)and 0.75 m/s(1.5 kts)at higher speeds. |
| Lat/Lon | (Int32)EncodedValue=Value*10^7 Default/Unknown:0,0 | (Double)Value=EncodedValue/10^7 |
| Vertical Speed | (Int8)EncodedValue=Value/0.5 | (Float)Value =EncodedValue*0.5 |
| Altitude | UInt16)EncodedValue=(Value +1000)/0.5 Unknown:-1000,encode as 0 | (Float)Value=(EncodedValue*0.5)-1000 Encoding Rationale:Eliminated unused negative integer space and increases precision to 1/2 m If decoded Value=-1000,then real value is unknown |
| Time Stamp | (UInt16)Encoded Value =Tenths of seconds since current hour | if Encoded Value>Tenths of seconds since the current hour at time of receipt, then ValueTenths =tenths of seconds since previous hour else ValueTenths =tenths of seconds since current hour Value =Curent UTC Date/Time(Hour)+ValueTenths(of seconds) This is the "Time of Applicability" |

A Encoded Value must be rounded to nearest Integer.

B If value decodes to 255,then an unknown value is being represented. CIf value decodes to 254.25,then speed is at least 254.25.

AUTHENTICATION MESSAGE (Tables 8 and 9)

5.4.5.9  Authentication Message  Type: 0x2,Static Periodicity,Optional*(see 5.3.2).

5.4.5.10 Authentication Message Overview:

5.4.5.11  The  Authentication  Message  defines  a  field  that provides  a means  for  authenticating the  identity  of the UA sending  the  message(as specified  in  Tables  8  and  9).An implementation could send an Auth Type of 0 to deterministically communicate that no Auth Data is intended to be sent. Alternatively,an  implementation  could simply  not  send  an Authentication message when there is no authentication data to send.Auth Types 1,2,and 3 represent standard signature options.Auth Type 4 is used to communicate that authentica-tion is provided by the Network Remote ID counterpart to the broadcast. Custom authentication implementations can be created using Auth Types A-F.The details of a custom implemen- tation are beyond the scope of this specification.

5.4.5.12  For  Auth  Types  1,2,and  3,the  standard  provides flexibility to allow a multitude of signature formats that are not specified in this specification.The intended implementation is that  an  agreed  upon  signature  format  for  each  Auth  Type required will be shared by both the signature encoding software and the verifier software.This specification specifies an API for a receiver (for example,a Remote ID Display Application)to communicate  with  a  verifier.(See  Annex  A1  for additional details.)

5.4.5.13 Authenticate Message Requirements:

5.4.5.14 If no authentication is used,and this message is still being sent(which is not required),the Auth Type shall be set to  0(BMG0120)and  the Signature  shall  be  empty.When  a signature is required,the signature produced by a UAS shall (BMG0130)be  encoded  to  match  the  signature  format  expected by the verifier.When UAS ID(1)or  Operator ID(2)is set as the AuthType,then the Message  Signature shall (BMG0140)include  the  corresponding  data  and  TimeStamp from  the  Authentication  message  in  the signature.If  the AuthType  is  set  to Message  Set(3),then  the  Signature  shall (BMG0150)include the concatenation (in message type order) of all other transmitted message types(excluding this Authentication  message),and  TimeStamp  from  this  Authentication message.

5.4.5.15  The Authentication Message Data Page field allows for Authentication Data sizes that may exceed the 24 bytes available  per  message.The Data  Page  shall(BMG0160)be incremented(starting from 0)for each additional message required to complete the oversized message.The Length field shall specify the exact length in bytes of the authentication payload data.Additional data may be  sent(for example: additional Forward  Error  Correction(FEC)data)after  the authentication payload data.If additional data is sent,it must begin with one(unsigned int8)byte indicating the length of the additional  data.The  LastPageIndex  value  must include  the authentication  payload data  and  any  additional  data.If  no additional data is sent,then the unused portion of the authentication data field shall be filled with nulls(which also indicates zero-length  of  “additional  data”).AuthType  3(Message  Set) shall(BMG0170)only be used when the transport media can send all of the pages together,such as Bluetooth 5 or Wi-Fi.If the AuthType is 4(Network Remote ID),then the Authentica- tion  Data/Signature(BMG0180)shall  be  empty(all  nulls).If the AuthType is 5(Specific Authentication Method),then the first byte of the first page of authentication data must be the Specific Authentication  Method Type(1-255)where  0 is reserved,1-224 is the registered type,and 225-255 are avail- able for private experimental use only.If using this method,the Specific Authentication Method Type  shall(BMG0185)be registered  through  the  registrar(see Annex  A5).Using  the experimental range is not considered “compliant”to this specification,but may be used for testing and experimentation.

TABLE 8 Authentication Message Page 0 Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations Example |
| --- | --- | --- | --- | --- |
| 1 | 1 | AuthType, Page Number | Bits [7..0][0000][0000] Auth Type:Bits [7..4] 0:None 1:UAS ID Signature 2:Operator ID Signature 3:Message Set Signature 4:Authentication Provided by Network Remote ID 5:Specific Authentication Method 6-9:Reserved for Spec A-F:Available for Private Use Data Page Number:Bits [3..0] | Up to 16 Types,Page number must be 0 |
| 2 | 1 | Last Page Index | Bits [7..0][0000][0000] Reserved:Bits [7..4] Last Page:Bits [3..0] Start at 0 | Up to 16 pages indexed Ox0-0xF |
| 3 | 1 | Length (bytes) | Total Data Length of the concatenation of all Authentication Data/Signature fields from all authentication pages | 0 to 255 bytes After the Length limit is reached,additional data(starting with 1 length byte)may be sent |
| 4 | 4 | Timestamp | 32 bit Unix Timestamp(UTC)in seconds since (epoch)00:00:0001/01/2019(to relate back to standard Unix timestamp,add 1546300800 to the common epoch of 00:00:0001/01/1970) | up to Last Page Index. 01/01/2019 to 01/19/1987(UTC) |
| 8 | 17 | Authentication Data/Signature | Opaque Authentication Data | 0-16 pages |

TABLE 9 Authentication Message Pages 1 through 15 Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | AuthType, Page Number | Bits [7..0][0000][0000] Auth Type:Bits [7..4] 0:None(not necessary to send if 0) 1:UAS ID Signature 2:Operator ID Signature 3:Message Set Signature 4:Authentication Provided by Network Remote ID 5:Specific Authentication Method 6-9:Reserved for Spec A-F:Available for Private Use Page number:Bits [3..0] | Up to 16 Types,Page numbers 1-15 |  |
| 2 | 23 | Authentication Data/Signature | Opaque Authenication Data(Signature) | 23 Bytes per additional page |  |

SELF-ID  MESSAGE (Table 10)

5.4.5.16 Self  ID Message Type:Ox3,Static Periodicity, Optional*(see  5.3.2).

5.4.5.17  The  Self-ID  Message  is  an  opportunity  for  the Remote Pilot to (optionally)declare their identity or purpose (intent)of the flight,or both.This message can serve to reduce the perceived threat  of a UA  flying  in  a  particular  area  or manner.For  example:to  put  neighbors  at  ease,a  realtor  may declare a “property photo shoot"of a client's house.This is a free-form(ASCII)text field.Also,this message can be used to add details about an emergency status or further details about the Operational Status field that is indicated in the Location/ Vector message.

TABLE 10 Self-ID Message Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Description Type | 0:Text Description 1:Emergency Description 2:Extended Status Description 3-200:Reserved 201-255:Available for private use | 0-255 | 0 |
| 2 | 23 | Description | ASCII Text.If numeric values exist,they shallbe expressed as a string of ASCll characters(padded with nulls) | 23 Bytes | DronesRus:Survey |

SYSTEM  MESSAGE (Table 11)

5.4.5.18 System  Message Type:Ox4,Static  Periodicity, Optional*(see 5.3.2).

5.4.5.19 The System Message contains general system in- formation including information about the Remote Pilot location and flight area.If the GCS has a dynamic location source (for  example,GNSS),then the  Operator Location  fields  shall (BMG0190)be the current location information of the GCS as acquired from the dynamic source.If the GCS cannot obtain dynamic location data,then the Operator Location fields shall (BMG0200)be the aircraft's takeoff location.Since this value generally does not change at the same rate as a UA location,the minimum update frequency shall (BMG0210)be the same as static messages.If a group of aircraft is being represented,the number  of  aircraft,radius of  flight area centered  on  the Location/Vector Message latitude/longitude,and group operations ceiling  and  floor  shall(BMG0220)be  expressed  in this message  using  the  Area  fields.If  one  or  more  UA  are non-equipped,the  Area fields shall(BMG0230)be used to declare (by means of broadcast messages compliant with this section)a volume of operation by a device external to the UA (such  as  a  ground  station)centered  on  the  Location/Vector Message latitude/longitude.If the value for one or more fields is unknown,that field shall(BMG0240)be filled as specified in Table  1.

TABLE 11 System Message Details

| Offset (Byte) | Length (Bytes) |  | Data Field |  | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Flags |  | Bits [7..0][00000000] Reserved:Bits [7..5] Classification Type:Bits [4..2] 0=Undeclared 1=European Union 2-7=Reserved Operator Location/Altitude source type:Bits [1-0] |  |  |  |
| 2 | 4 | Operator Latitude |  | 0=Take Off 1 =Dynamic 2=Fixed Latitude of Remote Pilot |  | Int signed deg*10^7(LE) | -11989298 |
| 6 | 4 | Operator Longitude |  | Longitude of Remote Pilot |  | Int signed deg*10^7(11 mm precision)(LE) | 48123987 |
| 10 | 2 | AreaCount |  | Number of aircraft in Area,group or formation(default 1) |  | Up to 65000 (LE) |  |
| 12 | 1 | Area Radius |  | Radius of cylindrical area of group or formation*10 m(defaut 0)centered on Location/Vector Message position |  | Up to 2.5 km |  |
| 13 | 2 | Area Ceiling |  | Group operations ceiling WGS-84 HAE (Alttude+1000 m)/0.5 |  | -1000-31767m(107503 ft) 16 bit UInt(LE) | 2021(enc)= 10.5m |
| 15 | 2 | Area Floor |  | Group operations floor WGS-84 HAE (Altitude+1000 m)/0.5 |  | -1000-31767m(107503 ft) 16 bit UInt(LE) | 2021(enc)= 10.5m |
| 17 | 1 | UA Classification |  | When Classification Type is 1, encode as below.Otherwise,set to 0 Category:Bits [7..4] Class:Bits [3..0] 0:Undefined 0:Undefined 1:Open 1:Class 0 2:Specific 2:Class 1 3:Certified 3:Class 2 4-15:Reserved 4:Class 3 5:Class 4 6:Class 5 7:Class 6 8-15:Reserved |  |  | Classification Open/Class 3 Encoded as Ox14 |
| 18 | 2 | Operator Altitude |  | Altitude of Remote Pilot WGS84-HAE (Altitude+1000 m)0.5 |  | -1000-31767m(107503f) 16 bit Ulnt(LE) | 2021(enc)= 10.5m |
| 20 | 4 | Timestamp |  | Time of applicability of Location Message expressed as a 32 bit Unix Timestamp(UTC)in seconds since (epoch)00:00:0001/01/2019(to relate back to standard Unix timestamp,add 1546300800 to the common epoch of 00:00:0001/01/1970) |  | 01/01/2019 to 01/19/2087 UInt32(LE) |  |
| 24 | 1 | Reserved |  |  |  |  |  |

OPERATOR ID MESSAGE (Table 12)

5.4.5.20 Operator ID Message Type:Ox5,Static Periodicity, Optional*(see 5.3.2).

5.4.5.21 The Operator ID Message contains the CAA issued Operator ID formatted as described in Table 1.

TABLE 12 Operator-ID Message Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Operator ID Type | 0:Operator ID 1-200:Reserved 201-255:Available for private use | 0-255 | 0 |
| 2 | 20 | Operator ID | ASCII Text.If numeric values exist,they shallbe expressed as a string of ASCIl characters(padded with nulls) | Up to 20 Bytes |  |
| 22 | 3 | Reserved | Reserved |  |  |

MESSAGE PACK (Table  13)

5.4.5.22  Message  Pack  Message  Type:0xF,Dynamic Peri- odicity if dynamic message in contents.The message pack is a payload that combines the other message types into a single (long)message  called  a  “message pack.”This is most appro- priate  when  using  transmit(such  as  BT5  or  Wi-Fi)schemes capable  of  sending  large frames.When  using  the  message pack,all  message  types  being  sent  shall(BMG0250)be  sent together in one or more message packs and shall be sent at a periodicity of at least the dynamic message rate in accordance with 5.4.4 for message packs that contain dynamic data.

TABLE 13 Message Pack Message Details

| Offset (Byte) | Length (Bytes) | Data Field | Details | Limitations | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Message Size | Size of single message or page in message pack. Set to Ox19(25). | 0x19(25) | 0x19(25 bytes) |
| 2 | 1 | No of Msgs in Pack (N) | Number of messages and pages(N)contained in message pack | Up to 9 | 5 |
| 3 | N*25 | Messages [hdr][msg][hdr][msg] [….] | Series of concatenated messages in message number order.Each message contained herein starts with a header as described in 5.4.5.4. | Up to 225 bytes |  |

5.4.6 Bluetooth Legacy(4.x  compatible)Transport Method:

5.4.6.1 Bluetooth 4.x(and newer)is widely deployed across  diverse  commonly  carried  handheld  devices  and  provides  mechanisms for low bandwidth beacons.The implementation  method  utilizes  the existing“advertising  beacon  messages” that  are  commonly  used  to  declare  a  device(such  as  a  headphone or mouse)available for pairing.Now that Bluetooth 5 has introduced an “Extended Advertising”method,Bluetooth 4.x method is called “Legacy Advertising.”

5.4.6.2  As  illustrated  in  Fig.4,  the  most  common  Wi-Fi channels that are generally pre programmed into routers are 1,6,and  11 because  they do not overlap.Bluetooth channels(as illustrated with the blue bars)are much narrower than Wi-Fi channels.Bluetooth uses three  different  beacon  channels  (in orange-37,38,and  39)to  broadcast  messages  to  non-specific endpoints(connectionless).Although  the  remaining  37  channels operate in the 2.4 GHz range,proximate to where Wi-Fi resides,the beacon  channels  are  generally  outside  of typical Wi-Fi traffic bands.

5.4.6.3 Beacon Messages—Bluetooth supports a “Broadcast Frame”to  transmit  on  the  beacon  channels  with  a  custom message length limit of 31 bytes.This leaves 25 bytes (after certain  header  info)available  for  Open  Drone  ID  messages. These broadcast messages shall (BB40010)be “uncoded”and conform to Bluetooth Core Specification 5.0,Volume 6,Part B, Sections 2.1 and 2.3.1.

5.4.6.4  Frame  Details—Legacy  Advertising  Frames  shall (BB40030)be encoded as illustrated below in Fig.5  and Table  14.

WiFi

Channel  1

0  1 2  3  4 5 6  7  8 9

1  5 6 9 6  2  9

Bluetooth  Channels

2402  Mhz-2480  Mhz

FIG.4 Bluetooth Channels

6 9

□Beacon Channels

WiFi

Channel 6

WiFi

Channel 11

√

3

4

4

4

0

0

8

8

8

2

2

2

2

2

2

2

2

2

2

3

3

3

3

3

3

5

3

3

5

3

1

1

1

1

1

1

1

1

| Legacy Advertisement Frame |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preamble (1 Byte) | AcC Addr (4Bytes) | PDU Hdr (2 Bytes) | AD Addr (6 Bytes) | AD Len,Type (2 Bytes) | 16-bit UUID | App Code | Counter (1 Byte) | Open Drone ID Message (25 Bytes) | CRC (3 Bytes) |
| 0xAA | 8E89BED6h | 20h,25h | <public> | 1Eh,16h | 0xFFFA | 0x0D | 0x00-0×FF | <message> | <calculated> |

| Open Drone ID Message |  |  |
| --- | --- | --- |
| Msg Type (4 bits) | Version (4 bits) | Message (24 Bytes) |
| 0×0-0xE | 0×0-0xF | <Open Drone ID message> |

FIG.5 Legacy Advertisement Frame

TABLE 14 Legacy Advertisement Frame Details

| Fied | Size | Value | Contents |  |  |
| --- | --- | --- | --- | --- | --- |
| Preamble | 1 | 0xAA | LE 1M Packet |  |  |
| Acc Address | 6 | 0x8E89BED6 | Broadcast Packet |  |  |
| PDU Hdr | 2 | 0x2025 | PDU Type 0x2 RFU 0 ChSel 0 |  | ADV_NONCONN_IND-Connectionless Advertisement Reserved Reserved |
|  |  |  | TxAdd RxAdd Len | 0 or 1 0 0x25 | Indicates AD Addr is HW Address or Random Address Reserved 37 Bytes |
| AD Addr | 6 | 0xXXXXXX | Unique Hardware Address of Bluetooth MAC |  |  |
| AD Info | 4 | 1Eh,16h,OxFFFA | Length Type Mfg Code | 0x1E 0×16 0xFFFa | 30 Bytes(excluding this field) Service DataA ASTM (Little Endian(FA,FF))B |
| AD App | 1 | 0x0D | Application Code:0x0D=Open Drone ID |  |  |
| Message Counter | 1 | 0xXX | A Message Counter,as defned in 5.4.4.2. |  |  |
| ODID Msg | 25 | <25 Bytes> | Open Drone ID Message |  |  |
| CRC | 3 | <calculated> | CRC Error Detection Data as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.1.1 |  |  |

A See https:/www.bluetooth.com/specifications/assigned-numbers/generic-access-profilel.

B See https://www.bluetooth.com/specifications/assigned-numbers/16-bit-uuids-for-sdos/

5.4.7 Bluetooth  5 Long Range  Transport Method

— Bluetooth 5 allows for several new features over Bluetooth 4.x. For the purpose  of this  application,the most  important new features are Long  Range  Mode(LE  Coded)and  Advertising Extensions (allowing for larger connectionless broadcasts).

Bluetooth 5 General Requirements

5.4.7.1 If implementing this specification using Bluetooth 5 Long Range, Legacy (ADV_NONCONN_IND) advertisements must(BB50010)be  sent,as  described  in 5.4.6,  for backwards compatibility with less capable receivers.Bluetooth 5  Extended  Advertisements(ADV EXT  IND+AUX  ADV IND)must(BB50020)be  sent  as  well  at  the  same  rate  as Dynamic  Data  (see  5.4.4 Update  Rates)and they must (BB50030)be  sent on a LE Coded (S=8)PHY.This will add Forward Error Correction(FEC)and can increase the range of the  advertisements  by  a  factor  of  4.These  messages  shall (BB50040)conform to Bluetooth Core  Specification 5.0, Volume  6,Part  B,Section  2.2(LE  Coded  PHY,S=8).

5.4.7.2 While Legacy Advertisements broadcast on the beacon channels 37,38,39, Bluetooth 5 adds Extended Advertising  that  allows  for  up  to  255  byte  advertisements  on  the "non-beacon"channels  by implementing a pointer in the primary  beacons  directing  the  receiver  to  read  from  the  secondary channel.

Bluetooth 5 Extended Advertisement Primary(Pointer) Packet

5.4.7.3 The Bluetooth 5 Extended Advertisement Primary Packet includes a pointer to the Secondary Packet as illustrated in  Figs.6 and 7. Therefore,the Primary  packet shall(BB50050)be  broadcast  through  all  3  beacon  channels,fol- lowed by the Secondary packet on the remaining channels.

5.4.7.4 The Pointer Frame shall (BB50060)be encoded as described in Fig.8 and Table 15.

工 D R

工 D R

Legacy Advertising

Channels 37,38,39

Bluetooth 5 Advertising Extensions Channels 37,38,39

Secondary Channel 0-36

FIG.6 Bluetooth Advertising

DATA

(Up to 255 Bytes)

DATA

(26B)

DATA

(26B)

DATA

(26B)

P T R

P T R

P T R

H D R

H D R

H D R

H D R

H D R

Ch 37

Pointer

Beacon Ch 38

Pointer Beacon

Ch 39

Pointer Beacon

Ch×(1-36)

Message Pack

Msg 0 Msg 1 Msg 2 Msg X. …

Header

FIG.7 Bluetooth Extended Advertisement

| Bluetooth 5 Long Range Advertisement Pointer Frame (LE Coded) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preamble (1 Byte) [Coded Phy] | AcC Addr (4 Bytes) | Cl (2 bits) [S=8] | TERM1 (3 bits) | PDU Hdr (2 Bytes) | Ext Hdr Len (6 Bits) | Adv Mode (2 Bits) non-scan non-conn W/Aux Ptr | Ext Header | Adv Data | CRC (3 Bytes) | Term2 (3 bits) |
| 0×3C | 0×8E89BED6 | 00b | <xxx>b | 0x7X06 | 0x18(6,0) |  | <6 bytes> | N/A | <calculated> | <xxx>b |

FIG.8 Bluetooth 5 Long Range Advertisement Primary Packet Frame

TABLE 15 Additional Primary Packet Frame Details

| Name | Size (Bytes) | Value | Value Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preamble | 1 | 0x3C | LE Coded PHY |  |  |  |  |  |  |
| Acc Addr | 6 | 0x8E89BED6 | Broadcast Packet |  |  |  |  |  |  |
| Cl | 2 bits | 00b | Coding Indication:FEC Block 2 is coded using S=8(longest range) |  |  |  |  |  |  |
| Term1 | 3 bits | xxxb | FEC Block 1 Termination as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.3.1 |  |  |  |  |  |  |
| PDU Hdr | 2 | 0x7X06 | Field PDU Type RFU ChSel TxAdd RxAdd Len | Bits 0111 0 0 0 or 1 0 00000101 | Hex 0x7 0x0 0x07 |  | Desc ADV_EXT_IND(Primary) Reserved Reserved HW or Random Address Reserved 7 Bytes |  |  |
| Ext Hdr Len, Adv Mode | 1 6 Bits 2 Bits | 0x18 | Bits [7..0]:[000110][00] Bits [7..2]:Ext Header Len:6 bytes Bits [1..0]:Adv Mode Ox0,Non-connectable,Non-scannable with Aux Pkt |  |  |  |  |  |  |
| Ext Hdr | 6 |  | Field Flags ADI Aux Ptr | Size 1 2 3 | Bits(binary) 00011000 |  |  | Hex 0x19 0x0000- 0x000F 0xXXXXXX | Desc Field Seection(ADI,Aux Ptr) Advertising DataID(12 bits)=0 Advertising Set ID(4 bits): Increment each time data changes cccccc=Channel a=clock accuracy 0=30 us offset multiplier dddddd=offset/delay 010=LE Coded Phy (See Table 16) |
|  |  |  |  |  | XXXX XXXX cccc dddd dddd | XXXX XXXX cca0 dddd d010 |  |  |  |
| Adv Data | N/A | 0 | Not Populated for this message |  |  |  |  |  |  |
| CRC | 3 |  | CRC Error Corection Data as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.1.1 |  |  |  |  |  |  |
| Term2 | 3 bits |  | FEC Block 2 Termination as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.3.1 |  |  |  |  |  |  |

Aux Ptr Field Details

5.4.7.5 The Aux Ptr Field in the Primary Packet shall (BB50070)be implemented in accordance with Bluetooth Core Specification  5.0,Volume 6,Part B,Section 2.3.4.5 with the following guidance in Table 16.

TABLE 16 Aux Pointer Field Details

| Channel Index Clock Accuracy(CA) Offset Units Aux Offset/Delay Aux PHY | Shall(BB50080)be calcuated using the following formula: Channel=(Current Channel+9)%36 This will ensure some entropy by hopping through the channels and spreading out the beacons to minimize the effects of external interference. 0:51-500 ppm 1:0-50 ppm 0:30 us This represents the time offset between sending the primary and the secondary packet.Since all three Primary Packets are sent prior to the secondary packet,the offset is diferent for each one.This offset should be calculated based on the guidance given in Bluetooth Core Specification 5.0. The following offsets may be used as guidance: Beacon 1:166 us Beacon 2:114 us Beacon 3:62 us These calculations are based on a Primary Packet time of 1552 us+a T_MAFS(minimum aux frame space)of 300 us divided by the offset multiplier unit of 30 us.The time of sending the current beacon +remaining beacons must(BB50090)be included.Thus, Beacon 1 includes the time of itself+2 more beacons+T_MAFS.offset is different for each one. 010:LE Coded Phy |
| --- | --- |

Bluetooth 5 Advertising Extension Secondary Packet

5.4.7.6  The secondary packet contains the actual informa- tion payload. This packet shall(BB50100)be encoded accord- ing to the Common Extended Advertising Format described in Bluetooth  Core  Specification  5.0,Volume  6,Part  B,Section 2.3.4 with the values included in  Fig.9 and Table  17.

5.4.7.7  Additionally this  packet  Adv Data  payload shall (BB50110)use  the  “Message  Pack”(Message  Type  0xF) format described in 5.4.5.22,Fig.9,  and  in Table 13. No  more than 9 messages  shall(BB50120)be included  in a  single Message  Pack.

| Bluetooth 5 Long Range Extended Advertisement Data Frame (LE Coded) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preamble (1 Byte) [Coded Phy] | AcC Addr (4 Bytes) | CI (2 bits) [S=8] | TERM1 (3 bits) | PDU Hdr (2 Bytes) | Ext Hdr Len (6 Bits) | Adv Mode (2 bits) non-scan Undirect(0) | Ext Header | Adv Data |  |  |  |  | CRC (3 Bytes) | Term2 (3 bits) |
|  |  |  |  |  |  |  |  | AD Len,Type (2 Bytes) | 16-bit UUID | App Codek | Counter (1 Byte) | ODID Msg |  |  |
| 0×3C | 0x8E89BED6 | 00b | <xxx>b | 0x7XXX | 0×24 |  | <9 Bytes> | XXh,16h | 0xFFFA | 0×0D | 0x00- OxFF | <3+N* 25 Bytes> | <calculated> | <xxx>b |

*N=Number ofMessage Types in Message Pack

| Open Drone ID Message Pack |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ms9 Type (4 bits) [MsgPk] | Version (4 bits) | Single msg Size (1 byte) | No of Msgs in Pack (N) | Message (Type 0) | Message (Type i) | … |
| OxF | 0x0-0×F | 0x19(25) | <1 Byte> | <25 Bytes> | <25 Bytes> | … |

| Open Drone ID Message |  |  |
| --- | --- | --- |
| Msg Type (4 bits) | Version (4 bits) | Message (24 Bytes) |
| 0x0-0xE | 0x0-0×F | <Open Drone ID Message> |

FIG.9 Bluetooth 5 Extended Advertising Secondary Packet

TABLE 17 Bluetooth 5 Extended Advertising Secondary Packet Details

| Name | Size | Value | Value Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preamble | 1 | 0x3C | LE Coded PHY |  |  |  |  |  |
| Acc Addr | 6 | 0x8E89BED6 | Broadcast Packet |  |  |  |  |  |
| CI | 2 bits | 00b | Coding Indication:FEC Block 2 is coded using S=8(longest range) |  |  |  |  |  |
| Term1 | 3 bits | xxxb | FEC Block 1 Termination as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.3.1 |  |  |  |  |  |
| PDU Hdr | 2 | 0x7XXX | Field PDU Type RFU ChSel TxAdd RxAdd Length | Bits 0111 0 0 0 or 1 0 XXXX XXXX | Hex 0x7 0x0 0xXX | Desc AUX_ADV_IND (Secondary) Reserved Reserved HW or Random Address Reserved 18+N*25 Bytes where N is the number of Messages in the Message Pack |  |  |
| Ext | 1 | 0x24 | Bits [7..0]:[001001][00] |  |  |  |  |  |
| Hdr Len, | 6 Bits |  | Bits [7..2]:Ext Header Len:9 |  |  |  |  |  |
| Adv Mode | 2 Bits |  | Bits [1..0]:Adv Mode Ox0,Non-connectable,Non-scannable |  |  |  |  |  |
| Ext Hdr | 9 4 | XXh,16h, | Field Flags AdvA ADI | Size 1 6 2 | Bits(binary) 00001001 <HW ADDR> XXXX XXXX XXxx yyyy |  | Hex 0x09 0xXXXXXX 0x0000-0xFFFF | Desc Field Selection(AdvA,ADI) Adv Address(HW or Random Addr) Advertising Data ID(12 bits)=0 Increment each time data changes Advertising Set ID(4 bits): Undefined(not used) |
|  |  |  | Length XX XX Bytes(excluding this field) |  |  |  |  |  |
| AD Info |  | 0xFFFA | Type 0x16 Service DataA 16-bit UUID 0xFFFA ASTM (Little Endian(FA,FF)) |  |  |  |  |  |
| App Code | 1 | 0x0D | Application Code:Ox0D=Open Drone ID |  |  |  |  |  |
| Message Counter | 1 | 0xXX | A Message Counter,as deined in 5.4.4.2. |  |  |  |  |  |
| ODID Msg | 0xXX |  | Open Drone ID Message Pack |  |  |  |  |  |
| CRC | 3 |  | CRC Error Correction Data as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.1.1 |  |  |  |  |  |
| Term 2 | 3bits |  | FEC Block 2 Termination as defined in Bluetooth Core Specification 5.0,Volume 6,Part B,Section 3.3.1 |  |  |  |  |  |

A See https:/www.bluetooth.com/specifications/assigned-numbers/generic-access-profilel. B See https://www.bluetooth.com/specifications/assigned-numbers/16-bit-uuids-for-sdos/

5.4.8 Wi-Fi NAN Transport Method:

5.4.8.1 As detailed in this specification, a connectionless broadcast mechanism  can be implemented using Wi-Fi  Management Frames encapsulating Open Drone ID messages. For  better interoperability with handheld device  SDKs,Messages shall(BWF0010)be  encoded  within  the Service Discovery Frame  based on  the Neighbor  Awareness Networking(NAN) Specification.11,5This  solution  does not  require  connecting to any specific  wireless network  since  (on the  receiver)it utilizes the mechanism  that  simply  listens  for  Wi-Fi  broadcasts  and makes  the data available for display.These  broadcasts are implemented on 2.4  GHz and optionally on  5 GHz Wi-Fi.

Wi-Fi Management Frames

5.4.8.2 For  UAS implementing this protocol broadcast frame,a “management”(type 0), "beacon" (subtype 8), and “action”(subtype 13)frame  as  prescribed by the IEEE 802.11-2016 Part 11 Wi-Fi specification⁷,5 shall(BWF0020)be encoded as NAN Service Discovery Frames as described in the NAN Specification. Additionally,the values shall(BWF0030) be filled as described in the NAN Service Discovery Frame Diagram in Fig.10 and NAN Service Discovery Frame Details in  Table  18.Beacon frames called NAN Synchronization beacons shall(BWF0032)be transmitted at 6Mbps inside NAN Discovery Windows (DWs) used for NAN timing synchronization.The values of each beacon frame field shall (BWF0034)be filled as described in Table 19.

| 802.11(Type 0,Subtype 13)Vendor Specific Public Action(NAN Service Discovery Frame) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category (1 Byte) | Action (1 Byte) | OUI (3 Bytes) (Wi-Fi Alliance) | OUI Type (1 Byte) | NAN Attribute(s) |
| 0x04-Public | 0×09-Vend Specific | 50-6F-9Ah | 0x13 | <15+N*25 Bytes> |

| NAN Service Descriptor Attribute |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute ID (1 Byte) | Length (2 Bytes) | Service ID (6 Bytes) | Instance ID (1 Byte) | Requestor Instance ID (1 Byte) | Service Control (1 Byte) | Service Info Length (1 Byte) | Servic Info |  |
|  |  |  |  |  |  |  | Message Counter (1 Byte) | Open Drone ID Message Pack |
| 0x3 | <14+N*25> | 88-69-19-9D-92-09h | 0x01 | 0x00 | 0x10 | <4+N*25> | 0x00-0xFF | <3+N*25 Bytes> |

*N=Number of Message Types in Message Pack

| Open Drone ID Message Pack |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MsgType (4 bits) [MsgPk] | Version (4 bits) | Single msg size (1 byte) | No of Msgs in Pack (N) | Message (Type 0) | Message (Type 1) | . … |
| 0xF | 0x0-0xF | 0x19(25) | <1 Byte> | <25 Bytes> | <25 Bytes> | … |

| Open Drone ID Message |  |  |
| --- | --- | --- |
| Msg Type (4 bits) | Version (4 bits) | Message (24 Bytes) |
| 0x1-0xF | 0x0-0xF | <Open Drone ID message> |

FIG.10 NAN Service Discovery Frame Diagram

Octets:2  2  6 0 or 6  0 or 6  0 or 2  0 or 6  0 or 2  0 or 4  variable 4

| Frame Control | Duration /ID | Address 1 | Address 2 | Address 3 | Sequence Control | Address 4 | QoS Control | HT Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FIG.11 MAC Header

(1)NAN Synchronization beacon frames shall(BWF0036) be followed by Vendor Specific Public Action frames encoded as NAN Service Discovery Frames(SDF)within the DWs.The work sequence of the Management frames is described in NAN Specification.SDF frames are  used  to carry Remote ID  messages.

5.4.8.3  All  message  types  being  sent  shall(BWF0040)be sent using message pack(s)described in 5.4.5.22,Fig.9, and in Table  13.

Wi-Fi NAN Cluster ID

5.4.8.4 The NAN Cluster ID is a MAC address that takes a value from 50-6F-9A-01-00-00 to 50-6F-9A-01-FF-FF and is carried in the Address 3 field(BSSID)of MAC header (Fig.11)  of the  NAN  Service  Discovery  Frames  and  NAN  Beacon Frames.The NAN Cluster ID is generally randomly chosen by the  device  that  initiates  the  NAN  Cluster.However,for  this implementation,when sending the NAN Synchronization Bea- con  frames,the  Cluster  ID(defined  in  NAN Specification) shall(BWF0050)be a static value of“50-6F-9A-01-00-FF”. This is to facilitate the receiver to get Remote ID messages from multiple UAS in parallel.

NAN Service Discovery Frame and Synchronization Beacon Frame Format

5.4.8.5 The NAN Service Discovery Frame(SDF)is a Vendor Specific Public Action frame as defined in [3.8]1,5with the Wi-Fi Alliance OUI and Wi-Fi Alliance OUI type indicating the NAN protocol.The format and the values for the NAN SDF are defined in  Fig.10  and Table  18.The mandatory NAN Service  Descriptor  Attribute  shall(BWF0060)be  included  in the NAN SDF frames.Please refer to NAN Specification Part 9 for detailed information.Service Descriptor Extension attribute(SDEA)is  mainly  used  to  ensure  that  the  receiver  can continuously receive and update the DRI messages carried by SDA.Please refer to NAN Specification for detailed information.The field value of Master Performance shall(BWF0070) be  0xFE  and  Random  Factor  shall(BWF0080)be  0xEA  to allow multiple receivers to receive DRI messages in parallel.

TABLE 18 NAN Service Discovery Frame Details

| Field Size | Value | Description |
| --- | --- | --- |
| Category ID 1 | 0x4 | IEEE 802.11 Public Action Frame |
| Action Field 1 | 0x9 | IEEE 802.11 Public Action Frame Vendor Specific |
| OUI 3 | 50-6F-9A | Wi-Fi Alliance Specific OUI |
| OUI Type 1 | 0x13 | Identifying the type and version of the NAN NAN Attributes |
| Attributes ID 1 | 0x3 | Ildentifies the type of NAN atribute (Service Descriptor attribute) |
| Length 2 | Variable | Length |
| Service ID 6 | 88-69-19 9D-92-09 | This is a mandatory field that contains the first 48 bits of the SHA-256 hash of the Service Name.A lower- case representation of the Service Name shall(BWF0070)be used to calculate the Service ID.The format of the Service ID field shall(BWF0080)be as defined as the Service Hash in"Wi-Fi Peer-to-Peer Services Techncal Specification".The service name is "org.opendroneid.remoteid".Hash:88-69-19-9D-92-09h. |
| Instance ID | 1 | Publish_ID or Subscribe_ID,Value of zero is reserved |
| Requestor ID 1 | 0x00 | Instance ID from the frame that triggered the transmission if available,otherwise set to 0x00. |
| 1 | 0x10 | Mandatory field that defines the Service Control bitmap as defined below. Bit 0-1:Identifies the Service Control Type.The value shallbe set to "00:" 00:Publish |
| Service Control |  | 01:Subscribe 10:Follow up 11:Reserved Bit 2-3:“00” Bit 4:The value shall be set to“1”and present Service Info field is found in the Service Descriptor attribute |
| Service Info Len 1 | 4+(N*25) | Mandatory field set to the length of Service Info |
| Service Info 4+(N*25) | Variable | Mandatory field that carries the Open Drone ID message pack and with a capacity of up to 255 bytes. Message Counter 0x00-0xFF:A Message Counter,as defined in 5.4.4.2. Open Drone ID MessagePack Open Drone ID Message Pack |
| Attribute ID 1 | 0x0E | Service Descriptor Extension attribute |
| Length 2 | 0x0004 | Length of the following fields in the attribute |
| Instance ID 1 | 0x01 | Publish_ID or Subscribe_ID,Value of zero is reserved |
| Control 2 | 0x0200 | SDEA Information Control field |
| Service Update 1 Indicator | 0x00~0xFF | Similar to a message counter.Monotonically increasing value indicating the current version of the service- specific information corresponding to the publish instance,which may be conveyed by publish messages |

TABLE 19 NAN Synchronization Beacon Frame Details

| Field | Size | Value | Description |
| --- | --- | --- | --- |
| Element ID | 1 | 0xDD | IEEE 802.11 Beacon Frame Vendor Specific |
| Length | 1 | 0x19 |  |
| OUI | 3 | 50-6F-9A | Wi-Fi Alliance Specific OUI |
| OUI Type | 1 | 0x13 | Identifying the type and version of the NAN NAN Attributes |
| Attribute ID Length | 1 | 0x0 | Master Indication attribute |
|  | 2 | 0x2 | Length of the following fields in the attribute |
| Master Performance | 1 | 0xFE | Information that is used to indicate a NAN Device's preference to serve as the role of Master,with a larger value indicating a higher preference.This value shall be 0xFE such that each UAS performs as a Master Cluster. |
| Random Factor | 1 | 0xEA | A random number selected by the sending NAN Device.This value shall be fixed to 0xEA rather than a random number. |
| Attribute ID | 1 | 0x1 | Cluster Attribute |
| Length | 2 | 0x0D | Length of the following fields in the attribute |
| Anchor Master Information | 13 | Variable | Information about the Cluster's Anchor Master,please refer to NAN Specification Part 9.5.2 for detailed information.For example,assuming drone MAC address is 62-62-1F-AF-D8-FA,Master Performance is FE,Random Factor is EA,the value of Anchor Master Information is 62621FAFD8FAEAFE0000oo0000.It consists of the following subfields: Anchor Master Rank:62621FAFD8FAEAFE Hop Count to Anchor Master:00 Anchor Master Beacon Transmission Time:00000000 |

Wi-Fi Operating Channels

5.4.8.6  In  order  to  allow  operation  of  NAN  Discovery, broadcasting shall(BWF0090)operate  in channel  6(2.437 GHz)in  the  2.4  GHz  frequency  band  and  may  optionally operate in channel  149(5.745  GHz)in the  5 GHz band.

5.4.8.7 The transmission interval of the consecutive NAN Service  Discovery  frames  shall(BWF0100)meet  the  requirements of the update rate for dynamic or static messages.

5.4.9 Wi-Fi Beacon Transport Method:

5.4.9.1 The Remote ID Wi-Fi Beacon Transport method uses standard SSID beacon frames and adds additional payload to these frames using what's called a Vendor Specific Tag or Vendor Specific Information Element (IE). In  implementing this method,either a single channel (channel 6)or any beacon channels shall(BWFB0010)be used.

5.4.9.2 For UAS implementing this protocol,a standard 802.11 Beacon frame,which is a“management”(type O, “beacon”(subtype 8),frame as prescribed by the IEEE 802.11-2016 Part 11 specification shall (BWFB0020) encode the Remote ID message pack as a vendor-specific information element IE221(using OUI:FA-0B-BC,Vendor Type:Ox0D) payload as described in Fig.12  and Table  20.

| Vendor Specific Tag(IE 221) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Length (1 Byte) | OUI 3 Bytes | Vend Type (1 Byte) | Message Counter(1 Byte) | Adv Data |
| <8+N*25> | FA-0B-BC | 0x0D | 0x00-0xFF | <3+N*25 Bytes> |

*N =Number of Message Types in  Message  Pack

| Open Drone ID Message Pack |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MsgType (4 bits) [MsgPk] | Version (4 bits) | Single msg size (1 byte) | No of Msgs in Pack (N) | Message (Type 0) | Message (Type 1) | …… |
| 0xF | 0x0-0xF | 0x19(25) | <1 Byte> | <25Bytes> | <25Bytes> | … |

| Open Drone ID Message |  |  |
| --- | --- | --- |
| Msg Type (4 bits) | Version (4 bits) | Message (24 Bytes) |
| 0x0-0xF | 0x0-0xF | <Open Drone ID message> |

FIG.12 Vendor Specific Information Element

TABLE 20 Wi-Fi Beacon Frame Details

| Field | Size | Value | Description |
| --- | --- | --- | --- |
| Length | 1 | 8+N*25 | Vendor Specific Tag Length |
| OUI | 3 | FA-0B-BC | ASD-STAN(for Harmonizaton) |
| Vendor Type | 1 | 0x0D | Open Drone ID |
| Message Counter | 1 | 0x00-0xFF | A Message Counter,as defined in 5.4.4.2. |
| Message Pack | 3+(N*25) | Variable | Mandatory field that caries the Open Drone ID message pack and with a capacity of up to 250 bytes. |

Wi-Fi Beacon Operating Channels

5.4.9.3 The broadcast message shall (BWFB0030)be transmitted on either:

(a)Any Wi-Fi channel of the 2400MHz to 2483MHz band;and according to the beacon frame standard as defined in the IEEE 802.11 standard.In this case,the message broadcast interval shall (BWFB0040) be <= 200 TU.

(b)Or Any Wi-Fi channel of the 5150 MHz to 5895 MHz band;and according to the beacon frame standard as defined in the IEEE 802.11 standard.In this case,the message broadcast interval  shall(BWFB0050)be<=200  TU.

(c)Or a single“social”Wi-Fi Channel on 2.4G band (Channel 6)or 5 GHz band(Channel 149).The broadcast message broadcast interval shall (BWFB0060)meet the mini- mum requirements of the update rate for dynamic and static messages.

5.5  Network  Remote  ID:

5.5.1  This section  presents  requirements for  the four  Net- work Remote  ID  interfaces  and  associated  functions  identified in Fig.1  .The interfaces include:

5.5.1.1  Networked  UAS  to  Net-RID Service  Provider

5.5.1.2 Operator of Intent-Based Network Participant to Net-RID Service  Provider

5.5.1.3  Net-RID  Service  Provider  to  Net-RID  Display  Pro- vider

5.5.1.4  Net-RID  Display  Provider  to  Display  Client.

5.5.2  UAS to Network Remote ID  Service Provider Require- ments:

5.5.2.1  Context:

5.5.2.2  This  subsection  provides  requirements  for  a  Net- RID Service Provider with respect to data received by means of its  interface  with  Networked  UAS,as  illustrated  in  Fig.13.

5.5.2.3  This  specification  does  not  specify  an  API  for  the communications  interface  between  Networked  UAS  and  Net- RID Service  Providers.However,in order to  participate in Network  Remote  ID, the  interface  must enable  the  Net-RID Service  Provider  to  construct  messages  that  comply  with  the required  Remote  ID  data  fields  and  to  be  able  to  provide updated  location  data  at  the  required  periodicity.

5.5.2.4 Requirements:

(1)UAS shall(NET0010)authenticate with  Net-RID  Ser- vice  Providers  using  an  industry-standard  authentication mechanism.

NoTE 3—Authentication of the UAS to the Net-RID Service Provider  provides confidence in the Remote ID  identity provided by the UAS. OAuth 2.0 or later is an example of an industry-standard authentication  mechanism by attestation,for those OAuth2 identity providers that require  authentication to obtain credentials.

(2)Communication between UAS  and Net-RID  Service Providers shall(NET0020)be encrypted using  an  industry- standard  encryption  mechanism  with  a  minimum  encryption strength  of  128  bits.

NoTE 4—This requirement is intended to address both integrity and confidentiality of Remote ID data in transit. TLS is an example of an industry-standard authenticated encryption mechanism.

(3)The Net-RID  Service Provider  shall(NET0030)notify the operator of a Networked UAS if the UAS is not providing necessary  data  to  participate  in  Network  Remote  ID.

NoTE 5—A timely notification is intended,but the manner in which the notification is delivered is at the implementer's discretion,and could use electronic or manual methods;therefore,a specific timing requirement for the notification is not specified.An implementer must describe the method(s)used in the product test report.

Networked UAS

Direct from  Drone  or  GCS

Net-RID Service  Provider 	(USS)

FIG.13 UAS to Net-RID Service Provider Context

(1)Net-RID Service Providers that support Intent-Based Network Participants shall(NET0110)provide the ability for the operator of an Intent-Based Network Participant to submit, modify,or  delete  an  operation  plan.

NoTE 7—An operating plan for an Intent-Based network participant consists of the “Operating Area”fields in Table 1.This includes Operating Area Polygon,Type,Count,Floor,Ceiling,Start,and End.Because many of the Common Data Dictionary fields cannot be known in the case of an

Intent-Based network participant (for example,Track Direction,Altitude,

Speed),appropriate null values should be provided.

(2)Net-RID Service Providers  shall(NET0120)require authentication  of  operators  using  an  industry-standard  authen- tication  mechanism  when  operation  plans  are  submitted  for Intent-Based Network Participants.

NoTE 8—Authentication of the operator of an Intent-Based Network Participant provides confidence in the validity of an operation plan.OAuth 2.0 or later is an example of an industry-standard authentication mechanism,for those OAuth2 identity providers that require authentica- tion to obtain credentials.

(3)Communications between the Intent-Based Network Participants  operator  interface  and  a  Net-RID  Services  Pro- vider  shall  (NET0130)be  encrypted  using  an  industry-standard encryption mechanism with a minimum encryption  strength  of 128  bits.

NoTE 9—This requirement is intended to address both integrity and confidentiality of Remote ID data in transit.TLS is an example of an industry-standard authenticated encryption mechanism.

5.5.4 Net-RID Service Provider to Net-RID Display Pro- vider  Requirements:

Context

5.5.4.1 This section provides  requirements governing  the interactions  between  Net-RID  Service  Providers  and  Net-RID Display Providers,as illustrated in Fig.15.

5.5.4.2  The  role  of Net-RID  Display  Providers  is  to  aggre- gate information for all UAS operating in an area and provide it to client Remote ID display applications,such as a website or app.There  are  four  general  phases  relative  to  a  request:

(1)Discover  applicable  Net-RID  Service  Providers:a  Net- RID  Display  Provider  must  determine  the  Net-RID  Service Providers that are operating in an area requested by a Remote ID  display  application  end  user.Because  additional  Net-RID  Service Providers can come online or introduce operations into

(4)If  dynamic data(for example,position updates)are not being received  from  a  UAS  at  a  frequency  of  [NetMinUas

LocRefreshFrequency]Hz at least  [NetMinUasLocRefreshPer- centage]of the  time,the  Net-RID Service  Provider  shall (NET0040)notify the operator.

NoTE 6—The intention of this requirement is not to preclude intermit- tent loss of network connectivity,but to detect situations where a UAS appears never to provide position updates at the minimum required rate. When the Net-RID Service Provider detects that a UAS may not be able to comply with the reporting frequency requirement,it should notify the operator so that steps can be taken to resolve the situation.A timely notification is intended,but the manner in which the notification is delivered is at the implementer's discretion,and could use electronic or manual methods;therefore,a specific timing requirement for the notifi- cation is not specified.An implementer must describe the method(s)used in the product test report.

5.5.3  Operator  of Intent-Based  Network Participant  to  Net- RID  Service  Provider  Requirements:

Context

5.5.3.1 This section provides  requirements for Net-RID Service Providers  with  respect to support for Intent-Based Network  Participants,as  illustrated  in Fig.14.

5.5.3.2  An  Intent-Based  Network  Participant  is  a  UAS  that does not participate in network Remote  ID in real-time, meaning  neither  the  UA  nor  the  GCS  (if  present)is  network connected.However, the  operator  can  participate  in  network Remote ID by submitting an operation plan which identifies the location  and  schedule  for  the  operation,and  the  ID  of  the aircraft.The  operation plan  is used  as  a  volume-based  position report (that is,a position report whose uncertainty is defined by the  volume)during  the  planned  time  of  operation.

5.5.3.3  This  specification  does  not  require  that  all  Net-RID Service  Providers  must  support  Intent-Based  Network  Partici- pants;however,those  that  do  must  adhere  to  the  requirements in  this  section.

5.5.3.4  In  order  to  submit  operation  plans  for  Intent-Based Network Participants, it  is  expected  that  the  operator  will use an  app,web,or  a  direct  interface  with  the  Net-RID Service Provider.  This  specification  does  not  specify  the  details  of an API or user interface for this purpose,but does specify security requirements  and  the  minimum  data  that  must  be  collected.

5.5.3.5 Requirements:

Net-RID Service Provider

(USS)

Intent-Based

Operation  Reported

Preflight

FIG.14 Intent-Based Network Participant to Net-RID Service Provider

F3411-22a

Display Provider identifies

(discovers)all  relevant

services providers

Gets  flight  data  per  area

Gets  flight  detail  per  flight

Net-RID Display  Provider 	(USS)

Net-RID Service Provider 	(USS)

FIG.15 Net-RID Service Provider to Net-RID Display Provider

an  area  where  none  previously  existed,discovery  must  con- tinue while  an  end-user  is viewing  data  for  an  area.

(2)Data requests:once applicable Net-RID  Service Pro- viders are identified,the Net-RID  Display Provider  makes periodic  requests  to  the  applicable  Net-RID  Service  Providers for Remote ID data for UAS in the area.Two types of requests can  be  made:

(a)Position information(for example,lat,long,altitude, speed,direction)for  all  UAS  currently  in  the  area  or  that  were in the  area  within  the  [NetMaxNearRealTimeDataPeriod]. (Note that data for flights that will be in the area in the future should  not  be  provided.

(b)Detailed,non-position  information for  a particular UAS in the area(for example,UAS ID,operator location, operation description).

(3)Discard  data:all  UAS  Remote  ID  data  acquired  by  a Net-RID  Display  Provider  from  other  Net-RID Service  Pro- viders  must  be  discarded  within  a  specified  time  period  after the data is received.(A Net-RID Display Provider may itself be a  Net-RID  Service  Provider  and  is  not  required  to  delete  its own data.)Disposing of data when no longer needed  (in combination with limiting data access to only what is required for  display  areas  being  viewed  by  a  Net-RID  Display  Provid- er's  associated  Display  Applications)helps  protect  privacy  and sensitive  data  of  operators and,if  applicable,their customers.

5.5.4.3  The  data  exchange  requirements  between  Net-RID Service  Providers  and  Net-RID  Display  Providers  are  defined in Annex  A2.

5.5.4.4  Requirements:

(1)Net-RID  Display Providers  shall(NET0210)authenti- cate with  Net-RID Service Providers  using  an industry- standard authentication mechanism.

NoTE 10—Authentication of the Net-RID Display Providers provides confidence that Remote ID data requests originate from the purported source.OAuth 2.0 or later is an example of an industry-standard authentication mechanism,for those OAuth2 identity providers that require authentication to obtain credentials.

(2)Communications between Net-RID Display Providers and  Net-RID Services  providers shall(NET0220)be  encrypted

using  an  industry-standard  encryption mechanism with  a mini- mum  encryption  strength  of  128  bits.

NoTE 11—This requirement is intended to address both integrity and confidentiality of Remote ID data in transit.TLS is an example of an industry-standard authenticated encryption mechanism.

(3)A  Net-RID  Display  Provider shall (NET0230)request Remote  ID  data  from  a  Net-RID Service  Provider  only  for areas  in  which  end  users  are  currently  requesting  information by  means  of the  Remote  ID  Display  Applications  services.

(4)A Net-RID  Display  Provider shall(NET0240)request relevant UAS Remote ID information from a Net-RID  Service Provider  by  specifying  a  rectangular  area  with  a  diagonal  no greater  than  [NetMaxDisplayAreaDiagonal]km.

(5)A  Net-RID Service Provider shall (NET0250)provide an  error code  or message  and no Remote  ID  data in response to a request when the diagonal  specifying the rectangular area is greater than  [NetMaxDisplayAreaDiagonal]km.

(6)When  the  diagonal  for  the  requested  area  is  within  the limit,the Net-RID Service Provider(s) shall(NET0260)pro-

vide relevant Remote ID data for the requested rectangular area in  no  more  than  [NetSpDataResponseTime95thPercentile]sec- onds  95%  of  the time  and in

[NetSpDataResponseTime99thPercentile]99%of the  time. Relevant  data  includes  information  consistent  with  the com- mon  data  dictionary  described  in 5.3  and,if  requested by  the Net-RID  Display  Provider,near-real-time  position  information for UAs that are currently in the requested area or that were in the  area up  to [NetMaxNearRealTimeDataPeriod]seconds prior,including Intent-Based Network Participants.

NoTE 12—

(a)Relevant data does not include information for flights not yet active in the requested area such as a flight that is scheduled to depart in the near future.

(b)Networked UAS provide real-time,discrete positions information for their flights whereas Intent-Based participants provide static volume information describing the area of operation for their flights.Relevant data only includes the type of information that is appropriate for a given flight; volume information should not be provided by a Net-RID Service Provider for a Networked UAS,and discrete position information should  not be provided for an Intent-Based participant.

(c)The response time requirements apply both to requests from a Net-RID Display Provider for position information for all relevant UAS in an area,as well as for the detailed information for an individual UAS.

(d)Providing near-real-time position information if requested is in- tended to enable an implementation not to be forced to resend the same data a Net-RID Display Provider already has.For example,the Net-RID Display Provider could request the near-real-time information the first time it requests data for an area,and thereafter it would not need to be requested and resent.

(e)For simplicity,this requirement is written assuming a pull approach, meaning the Net-RID Display Provider requests data from the Net-RID Service Provider on a periodic basis,such as once per second.A subscription-based implementation can also meet the intent of this requirement  as long as the push interval aligns with the NetSpDataResponse95thPercentile and 99thPercentile response times.For example,if the 95th Percentile response time is 1 s,the subscription push rate must be once per second 95%of the time.

(7)For each applicable UA,near-real-time position infor- mation shall(NET0270)include:

(a)All  position  reports  in  the  requested  area  up  to NetMaxNearRealTimeDataPeriod]in the past.

(b)For each time a UA entered the requested area during [NetMaxNearRealTimeDataPeriod],the  last  position  report received outside of the request area.

(c)For each time a UA exits the requested area during [NetMaxNearRealTimeDataPeriod],the  first  position  report received outside of the requested area.

NoTE 13—Item(a)is intended to allow a display application to show UAs that were in a user-requested area within a short period of time during which the user may have been retrieving a hand-held device and starting an app.Items(b)and(c)are intended to allow a display application to show approximately where a UA entered or exited the requested area.

(8)If  a  networked  UAS  temporarily  loses  network connectivity,a  Net-RID Service Provider may(NET0280) derive  and  supply  location  information  from  UAS operation plan extrapolation to Net-RID Display Providers until network connectivity is reestablished and updated location information

is received from the UAS.

(9)A Net-RID  Service provider  shall(NET0290)not provide  extrapolated  location  information  to  Net-RID  Display Providers  for  a  UAS  if  network  connectivity  with  the  UAS exists  and  location  information  is  being  received.

NoTE 14—The intention is that extrapolation only be used when necessary.

(10)The  Net-RID Service  Provider shall(NET0300)in-form Net-RID Display Providers when flight plan extrapolation is being used to  supply position  information  for  a  UAS.

(11) When flight plan extrapolation is used to  supply position  information  for  a  UAS.the  Net-RID  Service  Provider shall(NET0310)characterize  the  accuracy  of  the extrapolated location data using the Vertical Accuracy and Horizontal Accuracy of Position data fields.

(12)If a  networked  UAS  loses  connectivity  and  the  asso- ciated Net-RID  Service  Provider  is  unable  to provide  extrapo- lated  position  data,  the  Net-RID  Service  Provider  shall (NET0320)provide  to  a  requesting  Net-RID  Display  Provider the  most  recent  position  report  and  an  indication  that  current data  is not being received.

(13)Net-RID Display  Providers shall(NET0330)retain data  obtained  from  Net-RID Service  Providers  for  no  longer than  [NetDpMaxDataRetentionPeriod]seconds.This  require- ment  does  not apply  in  the case  where a  Net-RID  Display Provider  is  also  a  Net-RID Service  provider;that  is,a  USS performing both roles  is not required to  delete  its  own  data.

5.5.4.5  In  addition  to  the  performance  requirements  pro- vided  above,Net-RID  Service  Providers  and  Net-RID  Display Providers must(NET0340)support  the  minimum APIs  and associated  requirements  defined  in  Annex  A2.

5.5.5 Net-RID Display Provider to Display Application Requirements:

Context

5.5.5.1  This section  provides  requirements governing  the interactions between Remote ID Display Applications and Net-RID  Display  Providers,as  illustrated  in Fig.16.

5.5.5.2  This specification does  not impose authentication requirements  on  the  communications  interface  between  Re- mote ID Display Applications and Net-RID Display Providers.

5.5.5.3  This  specification  neither  specifies  the  API  between display applications and Net-RID Display Providers,nor in- cludes  requirements  pertaining  to  the  user  interface  provided  by  a  Remote  Identification  Display  Application.(As  noted  in

4.3 ,Remote  Identification  Display Applications  are  outside  the scope of this specification.)However,this  specification does levy performance-based requirements  on Net-RID  Display Providers  and  their  interface  with  Remote  ID  Display  Appli- cations.

5.5.5.4  This  specification  is  intended  to  provide  the  logical equivalent of obtaining the license plate for a car observed by an  interested  party. To  enable  that  objective  but  enforce  the privacy principle of only  sharing  data that needs to be  shared and to discourage the use of Remote Identification as a means for  ongoing  surveillance  over  a  wide  area  or  to  mine patterns of life  for  users  of  drone  services.distance-based limitations

are  imposed  on  Remote  Identification  information  that  can  be obtained  as  illustrated in Fig.17  .

5.5.5.5  As  previously  discussed,no  data  is  provided  from  a Net-RID  Service  Provider  to  a  Net-RID  Display  Provider  for an area with a diagonal larger than NetMaxDisplayAreaDiago- nal.For  areas  less  than  or  equal  to  that  size,Net-RID  Display Providers enforce  two levels  of information provided  to Remote  ID  Display  Applications,based  on  NetDetailsMaxDis- playAreaDiagonal.

5.5.5.6 For  display areas larger than  NetDetailsMaxDisplayAreaDiagonal,position  information  for UAs  is  either  clustered  in  the  case  of multiple  UAs  in  close proximity,or  obfuscated  for  individual  UAs,using  a  circular position report with  a  count  of UAs.

5.5.5.7  For  display  areas  equal  to  or smaller  than NetDetailsMaxDisplayAreaDiagonal,position  information for individual  UAs  is  provided.

5.5.5.8 Two types of requests from a Display Application to a Net-RID Display Provider are  addressed in the requirements below,and they correspond to the two types  of  requests between Net-RID Display Providers  and Net-RID Service Providers  described  in 5.5.4. Position  information requests provide the location details for all UAS in the requested area, and includes items such as lat,long,altitude,speed,and track direction;or,in  the  case of  Intent-Based  participants,data describing the operation plan  such  as the  intended volume  for

Provides  aggregated  picture

across all Net-RID Service

providers  for

network-reported  UAS  for  an

area

F3411-22a

Bluetooth Bluetooth

Wifi

Network &Broadcast Integration

in Display App

Display App

Public,Law

Enforcement,

others

Net-RID Display  Provider 	(USS)

4.x

5.x

Users

FIG.16 Remote ID Display Applications to Net-RID Display Providers

Display area with diagonal<=

NetDetailsMaxDisplayAreaDiagonal:

Individual UAs provided

Display area with diagonal>NetMaxDisplayAreaDiagonal:no data provided

Display area with diagonal<=

NetMaxDisplayAreaDiagonal but >

NetDetailsMaxDisplayAreaDiagonal:

UAS grouped or obfuscated  ①

X……

3  X

FIG.17 Data Provision to Display Application Based on Display Area Size

the operation.Flight detail request provides additional data about  a  specific  UAS,such  as  the  UAS  ID,operator  location,

and  operation  description.(The  precise  data  for  each  interface is  fully  enumerated in the API provided in Annex A4.)

5.5.5.9  With  respect  to  UAs  that  were  operating  within  a user-requested display area within a short period of time (NetMaxNearRealTimeDataPeriod),Net-RID  Display  Provid- ers  can  provide  Remote  ID  Display  Applications  all  position information  for  the  UA  in  the  user-requested  display  area  for that period.While no requirements are levied on the user interface  for Remote  ID Display Applications,this  capability  is intended  to  allow  the  user  to  identify  a  UA  that  has  recently exited the  area  from  some  form  of interaction  with  its  history trail and without having to pan in multiple directions searching for  the  current  UA  location.

5.5.5.10  Requirements:

(1)Communications between Net-RID Display Providers and  Remote  ID Display  Applications  shall(NET0410)be encrypted  using an industry-standard encryption mechanism with  a  minimum  encryption  strength  of  128  bits.

NoTE 15—This requirement is intended to address both integrity and confidentiality of Remote ID data in transit.TLS is an example of an industry-standard authenticated encryption mechanism.

(2)A Net-RID Display  Provider shall(NET0420)respond to the initial request from a Remote ID Display Application for position data for all UAS in an area with a diagonal no greater than  [NetMaxDisplayAreaDiagonal] km in

[NetDpInitResponse95thPercentile]seconds  95%of the  time and in [NetDpInitResponse99thPercentile]seconds 99%of the time.

NoTE 16—The response time requirement for initial requests for an area is longer than subsequent requests to allow for the discovery process used by the Net-RID Display Provider to find and establish communications with the applicable Net-ID Service Providers.

(3)A  Net-RID  Display  Provider  shall  (NET0430)provide an  error code  or message  and no Remote  ID  data in response to a request from a Remote ID Display Application for an area with  a  diagonal  greater  than [NetMaxDisplayAreaDiagonal].

(4)A Net-RID Display  Provider shall(NET0440)respond to subsequent requests (that is,requests after the initial request) from a Remote ID Display Application for UAS Remote ID for an  area  previously  requested  within  the  past  [NetMinSession- Length]with  a diagonal no greater than  [NetMaxDispla- yAreaDiagonal]km  in [NetDpDataResponse95thPercentile] seconds  95% of the  time  and  in  [NetDpDataResponse99thPercentile]seconds 99%of the time.

NoTE 17—Conceptually,when a Display Application makes a request to a Net-RID Display Provider for an area,this begins a session.The session remains in effect as long as the user is viewing the area,during which the Display Application will continue to make data requests. NetMinSessionLength represents a Time To Live(TTL)mechanism for the session and allows the Net-RID Display provider to distinguish between initial requests(NET0420)and subsequent requests(NET0440, this requirement).When a new request is made (the initial request),a NetMinSessionLength timer is started.Ifa subsequent data request for the same area is received and the timer has not expired(the norm as long as the user continues to view the area),the Net-RID Display Provider is able to rely on the discovery process performed on the initial request and reset the timer on each request.If the timer has expired (implying the user has closed the display or moved the location),a request for data in the same

TEST METHODS

6.Scope

6.1  This section  outlines  the  test  methods  used  to  test compliance  with  the  Remote  ID standard.Broadcast and network Remote ID tests differ.

6.2 The test for broadcast Remote ID shall determine the power  level,latency,periodicity,and  protocol compliance.

6.3  The  test  for  network  Remote  ID  shall  determine  the latency,periodicity,reliability,protocol compliance,and  in- teroperability with other network Remote ID implementations.

7.Significance  and  Use

7.1  The  specification  is  intended  to  be  used  by  UAS manufacturers,UAS operators,Remote ID USSs,and CAAs to assess UAS or USS compliance,or both,with the Remote ID standard.

8.Hazards

8.1 Ensure that UAS are configured as to not cause harm to individual(s)conducting the test or third parties.

8.2 UAS that are powered or operational can present haz- ards.Ensure  that  propellers  are  removed  or  caged  during laboratory testing.

8.3 Field testing of UAS can present hazards.Take appro- priate safety precautions when field testing UAS.

8.4  When  testing  UAS  with  power  plants  or  lithium batteries,or  both,an  appropriate  fire  extinguisher  for  each application  should  be  within  reach.Participants  should  be made aware of the hazards of lithium batteries or flammable fuels,or both,and which fire extinguishers are appropriate for lithium or flammable fuel-based fires,or both.

9.Test Units

9.1 The UAS used in this test shall be mechanically and electrically equivalent to the actual flying configuration.The UAS must be operational and powered during testing.

10.Procedure

10.1 Compliance matrices to support a conformance deter- mination process are provided below.As part of determining compliance,implementers  must  validate  that  all  functional, performance,and  interoperability  requirements  are  met.Test results and a description of how requirements were validated must be documented in a product test report using the notes columns of the applicable compliance matrices and supplemen- tal documentation as needed.

10.2  Broadcast:

10.2.1 To determine compliance to any broadcast method,  compliance must be verified for all items in the“All Methods” section in addition to the applicable section for the broadcast  method  (that  is,Bluetooth  4.x,Bluetooth  5.x,Wi-Fi  NAN,or Wi-Fi Beacon).

10.2.2  To  create  a  Broadcast  Authentication  Verifier,see Annex A1 for the applicable compliance matrix.

10.2.3 All Methods Compliance Matrix(see Table 21).

10.2.4 Bluetooth 4.x  Compliance Matrix(see Table 22).

10.2.5 Bluetooth 5.x Compliance Matrix(see Table 23).

10.2.6 Wi-Fi NAN Compliance Matrix(see Table 24).

10.2.7 Wi-Fi Beacon Compliance Matrix(see Table  25).

10.3 Network:

10.3.1 Net-RID Service Providers shall (NET0500)provide a persistently supported test instance of their implementation for  use  by  Net-RID  Display  Providers  in  order  to  support interoperability testing.This test instance must use the current deployed version of the implementer's Net-RID software and provide a means for injection or generation of test data in a geographic test location.

10.3.2 Network Compliance Matrix(see Table 26).

10.3.3  Notes:

10.3.3.1 Requirements that are not applicable to a particular product(for example,UAS  to Net-RID  Service Provider requirements  for  an  implementation  that  is  exclusively  a Net-RID Display Provider)should be marked as  "NA"in the test report.

10.3.3.2 If a USS implementation includes a new or modi- fied DSS,an additional compliance matrix and test guidance are provided in A2.5.

11.Precision  and  Bias

11.1 All requirements that have necessary precision attri- butes have the required precision stated within the requirement itself.Otherwise,the  stated requirements must be met 95%of the time unless other local regulations apply.For values that declare their own precision(such as horizontal location, vertical location,speed,and timestamp),the precision required  shall be as required by local regulations.No information can be  presented on the bias of the test methods in this specification because no such requirements have an accepted reference value available.

12.Product Marking

12.1  UAS  that  are  capable  of  meeting  the  threshold  for compliance with this specification should be labeled Remote ID and Tracking capable.

12.2  UAS that Are Marked:

12.2.1“ASTM  F3411-22a-RID-N”compliant  must  comply with the Network Remote ID standard.

12.2.2“ASTM  F3411-22a-RID-B”compliant  must comply with the Broadcast Remote ID standard.

12.2.3“ASTM  F3411-22a-RID-NB”must comply  with both the Broadcast and Network Remote ID standards.

13.Packaging  and  Package  Marking

13.1 The packaging of UAS that are capable of meeting the threshold  for  compliance  with  this  specification  should  be labeled ID and tracking capable.

TABLE 21 All Methods Compliance Matrix

Requirement ID Section Reference Compliant  (Y/N)  Notes

BPWO010

BUR0010 BUR0020

BUR0030

BUR0040 BUR0050

BUR0060

BMG0010 BMG0020

BMGO030

BMG0040 BMG0050

BMG0060 BMG0065

BMG0070 BMG0080

BMGO090 BMG0100

BMG0110 BMG0120

BMG0130 BMG0140

BMG0150 BMG0160

BMG0170

BMG0180 BMG0185

BMG0190 BMG0200

BMG0210 BMG0220

BMG0230

BMG0240 BMG0250

5.4.3

5.4.4.1

5.4.4.1 5.4.4.1

5.4.4.1

5.4.4.2

5.4.4.2 5.4.5.2

5.4.5.3 5.4.5.3

5.4.5.3

5.4.5.3

5.4.5.3

5.4.5.3 5.4.5.4

5.4.5.6 5.4.5.6

5.4.5.8

5.4.5.8

5.4.5.14

5.4.5.14

5.4.5.14

5.4.5.14

5.4.5.15

5.4.5.15

5.4.5.15

5.4.5.15

5.4.5.19

5.4.5.19

5.4.5.19

5.4.5.19

5.4.5.19

5.4.5.19

5.4.5.22

TABLE 23 Bluetooth 5 x Compliance Matrix

TABLE 22 Bluetooth 4.x Compliance Matrix

| Requirement ID | Section Reference | Compliant | (Y/N) | Notes |
| --- | --- | --- | --- | --- |
| BB40010 BB40030 | 5.4.6.3 5.4.6.4 |  |  |  |

| Requirement | ID | Section Reference | . Compliant(Y/N) | Notes |
| --- | --- | --- | --- | --- |
| BB50010 |  | 5.4.7.1 |  |  |
| BB50020 |  | 5.4.7.1 |  |  |
| BB50030 |  | 5.4.7.1 |  |  |
| BB50040 |  | 5.4.7.1 |  |  |
| BB50050 |  | 5.4.7.3 |  |  |
| BB50060 |  | 5.4.7.4 |  |  |
| BB50070 |  | 5.4.7.5 |  |  |
| BB50080 |  | 5.4.7.5 |  |  |
| BB50090 |  | 5.4.7.5 |  |  |
| BB50100 |  | 5.4.7.6 |  |  |
| BB50110 |  | 5.4.7.7 |  |  |
| BB50120 5.4.7.7 |  |  |  |  |

13.2 Packaging of UAS that Are Marked:

13.2.1“ASTM  F3411-22a-RID-N”compliant  must  comply with the Network Remote ID standard.

13.2.2“ASTM  F3411-22a-RID-B”compliant  must  comply with the Broadcast Remote ID standard.

13.2.3“ASTM  F3411-22a-RID-NB”must  comply  with both  the  Broadcast  and  Network  Remote  ID  standards

14.Keywords

14.1  broadcast;network;remote ID;UAS traffic  manage- ment;unmanned  aircraft  systems

TABLE 24 Wi-Fi NAN Compliance Matrix

| Requirement | ID | Section Reference Compliant(Y/N) | Notes |
| --- | --- | --- | --- |
| BWF0010 |  | 5.4.8.1 |  |
| BWF0020 |  | 5.4.8.2 |  |
| BWF0030 |  | 5.4.8.2 |  |
| BWF0032 |  | 5.4.8.2 |  |
| BWF0034 |  | 5.4.8.2 |  |
| BWF0036 |  | 5.4.8.2(1) |  |
| BWF0040 |  | 5.4.8.3 |  |
| BWF0050 |  | 5.4.8.4 |  |
| BWF0060 |  | 5.4.8.5 |  |
| BWF0070 |  | 5.4.8.5 |  |
| BWF0080 |  | 5.4.8.5 |  |
| BWF0090 |  | 5.4.8.6 |  |
| BWF0100 5.4.8.7 |  |  |  |

TABLE 25 Wi-Fi Beacon Compliance Matrix

| Requirement | ID | Section Reference | Compliant(Y/N) | Notes |
| --- | --- | --- | --- | --- |
| BWFB0010 |  | 5.4.9.1 |  |  |
| BWFB0020 |  | 5.4.9.2 |  |  |
| BWFB0030 |  | 5.4.9.3 |  |  |
| BWFB0040 |  | 5.4.9.3 |  |  |
| BWFB0050 |  | 5.4.9.3 |  |  |
| BWFB0060 |  | 5.4.9.3 |  |  |

TABLE 26 Network Compliance Matrix

| Requirement | ID | Section Reference | Compliant(Y/N) | Notes |
| --- | --- | --- | --- | --- |
| NET0010 |  | 5.5.2.4 |  |  |
| NET0020 |  | 5.5.2.4 |  |  |
| NET0030 |  | 5.5.2.4 |  |  |
| NET0040 |  | 5.5.2.4 |  |  |
| NET0110 |  | 5.5.3.5 |  |  |
| NET0120 |  | 5.5.3.5 |  |  |
| NET0130 |  | 5.5.3.5 |  |  |
| NET0210 |  | 5.5.4.4 |  |  |
| NET0220 |  | 5.5.4.4 |  |  |
| NET0230 |  | 5.5.4.4 |  |  |
| NET0240 |  | 5.5.4.4 |  |  |
| NET0250 |  | 5.5.4.4 |  |  |
| NET0260 |  | 5.5.4.4 |  |  |
| NET0270 |  | 5.5.4.4 |  |  |
| NET0280 |  | 5.5.4.4 |  |  |
| NET0290 |  | 5.5.4.4 |  |  |
| NET0300 |  | 5.5.4.4 |  |  |
| NET0310 |  | 5.5.4.4 |  |  |
| NET0320 |  | 5.5.4.4 |  |  |
| NET0330 |  | 5.5.4.4 |  |  |
| NET0340 |  | 5.5.4.5 |  |  |
| NET0410 |  | 5.5.5.10 |  |  |
| NET0420 |  | 5.5.5.10 |  |  |
| NET0430 |  | 5.5.5.10 |  |  |
| NET0440 |  | 5.5.5.10 |  |  |
| NET0450 |  | 5.5.5.10 |  |  |
| NET0460 |  | 5.5.5.10 |  |  |
| NET0470 |  | 5.5.5.10 |  |  |
| NET0480 |  | 5.5.5.10 |  |  |
| NET0490 |  | 5.5.5.10 |  |  |
| NET0500 |  | 10.3.1 |  |  |
| NET0610 |  | A2.3.2 |  |  |
| NET0620 |  | A2.3.2 |  |  |
| NET0630 |  | A2.3.2 |  |  |
| NET0710 |  | A2.4.1 |  |  |
| NET0720 |  | A2.4.1 |  |  |
| NET0730 |  | A2.4.1 |  |  |
| NET0740 | A2.4.1 |  |  |  |

ANNEXES(Mandatory Information)

A1.BROADCAST AUTHENTICATION VERIFIER SERVICE

A1.1  Overview

A1.1.1 The basic design for broadcast authentication is for the signature specification to be defined by the verifier and the protocol to be defined by this ASTM specification.As such,the broadcast packet format is described in the broadcast section of the specification,and the compatible verifier service require- ments are described in this annex.(See  Fig.A1.1.)

A1.2 Verifier Service Requirements

A1.2.1  The  verifier  service shall(VF0010)implement  the signature verification algorithm that matches the agreed signa- ture format that will be sent by the broadcaster.

A1.2.2 The verifier service shall(VF0020)set up a web service  as  described  in  this  annex.

A1.2.3 The response time of the verifier shall(VF0030)be less than  [VfResponseTime95]seconds 95%of the time from receipt of verify request message to the transmission of the result.(See  Table A1.1.)

A1.2.4 The Verifier API  shall(VF0040)implement  a RESTful/JSON protocol on a web server with a TLS secured endpoint as described by the Verifier API OpenAPI Description in A1.4.

A1.2.5  The verifier  shall(VF0050)maintain  a  testing  end- point.

A1.2.6  Result  Codes—The  ResultCode  and  ResultString values shall (VF0060)be set  in the verifier response given the conditions in Table A1.2.

A1.3 Verifier API RESTful Interface Sample

A1.3.1 Below is an example of how a verification request may be sent.

A1.3.2 Sample Request—See Fig.A1.2.

A1.3.3 Request Enumerations:

AuthType:

(1)UASID

(2)OperatorlD

(3)MessageSet

(4)Network

(5)SpecificAuthMethod UASIDType:

(1)ANSI/CTA-2063-A Serial Number

(2)UAS Registration ID

(3)UTM UUID

(4)Session ID

A1.3.4 Sample Result—See Fig.A1.3.

A1.4 Verifier API OpenAPI 3.0 YAML Description

A1.4.1 The API YAML code can be found at the following URL:

https://github.com/opendroneid/authentication-verifier-api/tree/auth_1.1

A1.5 Authentication Verifier Compliance Matrix

A1.5.1  See Table A1.3.

Broadcast Auth Message

Aircraft

Validation Service API

Signature

Payload

[Defined by Validator]

[Defined by Validator]

Type:UASID,OPID,MsgSet

Type:UASID,OPID,MsgSet

Receiver (Phone)

(Optional)

Delegate

Validator

(Net Svc)

Not in

Standard

FIG.A1.1 Broadcast  Authentication  Verifier  Service  Overview

Valid?  Primary

Validator

√ (Net Svc)

optionai Delegate  URL

In

Standard

Signature

Payload

TABLE A1.1 Authentication Verifier Values

| Value Name | Value | UoM | Req Ref | Section | Ref |
| --- | --- | --- | --- | --- | --- |
| VfResponseTime95 | 1 | Second | VF0030t | A1.2. | 3 |
| 十Editorially corrected TABLE A1.2 Authentication Verifier Service ResultCodes and ResultStrings Code Result String Condition 0 VALID Signature and message is valid 1 INVALID Message is Invalid or not authentic 2 VALID BUT EXPIRED Message is Authentic and Key is Expired 3 UNKNOWN KEY Key cannot be found or validity cannot be determined 4 REVOKED Key has been revoked 5 UNKNOWN ID The ID sent cannot be found 6 <Delegate endpoint URL>/remoteidverify/ Delegate to alternate verifier endpoint URL(including"/remoteid/verify") |  |  |  |  |  |

POST https://EndpointURL/remoteid/verify/ { "AuthType":1, "UASID":"N.12345", "UASIDType":2, "operatorID":"D.OP55544", "MessageSet":"base64(concatenated all other messages)", "Authsignature":"92429d82a41e930486c6de5ebda9602d55c39986", "TimeStamp":"2016-08-29T09:12:33.001Z" {

FIG.A1.2 Sample Request

{ "ResultCode":0, "ResultString":"Result String or Delegate Endpoint URL" }

FIG.A1.3 Sample Result

TABLE A1.3 Authentication Verifier Compliance Matrix

| Requirement ID | Section Reference | Compliant(Y/N) | Notes |
| --- | --- | --- | --- |
| VF0010 | A1.2.1 |  |  |
| VF0020 | A1.2.2 |  |  |
| VF0030 | A1.2.3 |  |  |
| VF0040 | A1.2.4 |  |  |
| VF0050 | A1.2.5 |  |  |
| VF0060 | A1.2.6 |  |  |

A2.NETWORK REMOTE ID INTEROPERABILITY REQUIREMENTS,APIs,AND TESTING

A2.1  This  annex  defines  requirements,APIs,and  testing for network  Remote  ID  interoperability  between  Net-RID  Service Providers and  Net-RID Display Providers.A YAML (OpenAPI)description  of the  APIs  is provided  in  Annex  A4.

A2.2  Network  Remote ID  Interoperability  Overview

A2.2.1  Fig.A2.1 summarizes the interfaces and  flow  of information  for  network  Remote  ID  interoperability.

A2.2.2  As  discussed  in  the  USS  Interoperability overview provided  in  4.8,an instance of discoverable information is referred  to  as  an  entity.The  discoverable  entity  defined  for Network  Remote  ID  is  an  Identification Service  Area  (ISA). This is a 4-D volume(a volume defined in x,y,and z plus time limits)that  corresponds  to  an  area  where a  Net-RID Service Provider  typically  has one  or  more  UAS operating,though  a Net-RID  Service  Provider  is  not  required  to  have an  active UAS in a given ISA at all times.

A2.2.3 Net-RID  Service  Providers  have  flexibility  in  defin- ing ISAs.Representative choices include:

A2.2.3.1  An  area  corresponding  to  a  single  operation-this could be used when a USS does not have a high frequency or concentration  of flights  in  a  given  area.

A2.2.3.2  An  area  corresponding  to  a  group  of  operations- this could be used when a USS has frequent flights in a fixed  location,as  might  be  the  case  with  delivery  services  or  urban air mobility.In this  case,a  single  ISA  reduces  the  overhead  of creating and managing ISAs for each individual flight.(A tradeoff for the USS is that it may receive requests for a portion of the ISA where operations do not exist at a particular point in time.)

A2.2.3.3 An area corresponding to a portion of an operation—this  could  be  used  for  a  flight  that  travels  a  long distance where the USS does not want to process data requests for portions of the flight that the UAS has left or has not yet reached.

A2.2.4  This  choice  of  these  or  other  options  is  left  to  the Net-RID  Service  Provider.The  standard  requires  only  that  all UAS  flights  operating  in  conjunction  with  a  Net-RID  Service provider  are  included  within  at  least  one  ISA.

A2.2.5 The interfaces  and  flow  of information  in  Fig.A2.1 is  as  follows(corresponding  to  the  numbered  interfaces  in  the figure):

A2.2.5.1  Net-RID  Service  Providers  make  the  UAS  opera- tions they support discoverable by writing an ISA entity summary  to  the  DSS.The  entity summaries  are  mapped  into the  airspace  representation  encapsulated  by  the  DSS.The  DSS API also  supports modifying,deleting  and retrieving the details of  ISA.

A2.2.5.2  Net-RID  Display  Providers  discover  the  Net-RID Service  Providers  with  which  communication  is  required  by querying the DSS for an area of interest (that is,an area being viewed  in  one  or  more  Remote  ID  Display  Applications)and establishing  a  subscription  for  notification  of  any  subsequent

Discovery &Synchronization Service

DSS

1)Make  Discoverable  Representation 2  )Discover

●PUT/DEL

Identification Service  Area

Entity  Summaries

Subscription Notifications

3 )Get Remote ID Data

Net-RID

Display

Providers

Data Exchange Protocol

●PUT/GET/DEL

Subscriptions for Identification

Services  Areas

Net-RID  Service Providers

Airspace

4

FIG.A2.1 Network Remote ID Interoperability Overview

F3411-22a

changes.(A  subscription  interface  rather  than a  polling inter- face  is  used  to  avoid  inefficient  usage  of the  DSS,which  is  a shared UTM  ecosystem  resource;that  is,it  would  be  inefficient to repeat the  complete  discovery  process  once  a  second  for  a large  number  of areas  of interest.)The  DSS  maps  the  area  of interest into the airspace representation to  find the intersecting cells with ISAs(if  any).For each intersecting ISA,the DSS returns a URLused by the Net-RID Display Provider to obtain Remote ID data from the Net-RID  Service Provider that created  the  ISA(step  3).

A2.2.5.3 Given the URLs for each ISA,the Net-RID Display  Provider  polls  the  applicable  Net-RID  Service  Pro- vider  to  obtain  the  relevant  Remote  ID  data  for  UAS  in  the area.This  peer-to-peer  interface  has  two  parts,one  to  obtain position information for all UAS the Net-RID Service provider has  in  the  requested  area,and  a second  to  obtain  detailed  information  for  a  particular  flight  (for  example,the  UAS  ID, operation  description,operator  location,etc.).

A2.2.5.4 If a Net-RID  Service Provider adds or modifies an ISA in a location that intersects a DSS airspace representation cell  with  a  Net-RID  Display  Provider  subscription,the  DSS informs  the  Net-RID  Service  Provider  and  provides  the  URL necessary to contact the subscribing Net-RID Display Provider. The Net-RID Service Provider calls the Net-RID Display Provider  and  provides  the  URL  the  Display  Provider  uses  to poll  the  Service  Provider.

A2.2.6  Detailed  requirements  and  APIs  for  these  interfaces are  provided  below.

A2.3 USS-DSS Interfaces

A2.3.1  DSS  Implementation Requirements:

A2.3.1.1  The  requirements  in  this  section  apply  exclusively to  implementers  of  a DSS.Note  that  specific  response  time requirements  for DSS  functions  are  omitted  because  DSSs  are not  tested  in  isolation,but  rather  in  conjunction  with  a  USS. DSS  processing  time  must  support  a  USS  meeting  applicable response  time  requirements.

(1)A  DSS instance shall(DSS0010)authenticate  USSs using  an  industry-standard  authentication  mechanism.

NoTE A2.1—OAuth 2.0 or later is an example of an industry-standard authentication mechanism by attestation,for those OAuth2 identity providers that require authentication to obtain credentials.

(2)Communication between a USS  and DSS instances shall(DSS0020)be encrypted using  an industry-standard encryption mechanism with a minimum encryption  strength  of 128  bits.

NoTE A2.2—This requirement is intended to address both integrity and confidentiality of Remote ID data in transit.TLS is an example of an industry-standard authenticated encryption mechanism.

(3)A DSS  implementation  shall(DSSO030)minimally include  the  following  interfaces  for  use  by  Net-RID Service Providers  and  Display  Providers,in  accordance  with  the  DSS portion  of the  OpenAPI  specification  presented  in Annex A4:

(a)PUT  Identification Service Area—this interface en- ables  a Net-RID  Service  Provider  to  create  or  modify  an  ISA entity  summary  in  the  DSS.

(b)DELETE Identification Service Area—this  interface enables  a Net-RID  Service  Provider  to  delete  an  existing  ISA

entity  summary  from  the  DSS.(A  Net-RID Service  Provider can  only  delete  ISAs  it  created.)

(c)PUT  Subscription—this interface  creates  a  subscrip- tion for  new or modified ISAs  within a 4-D  volume,and returns the intersecting ISAs resident in the DSS at the time of the  call.

(d)DELETE  Subscription—this  interface  enables  a  Net- RID  Display  Provider  to  delete  a  subscription  from  the  DSS.  (A  Net-RID  Display  Provider  can  only  delete  subscriptions  it created.)

(e)GET  Subscription—this interface enables a  Net-RID Display  Provider  to  retrieve  the  details  of  a  specific  existing  subscription  to  verify  its  existence  and  composition.(A  Net- RID  Display  Provider  can  only  retrieve subscriptions  it cre- ated.)

(f)GET  Subscriptions—this  interface  enables  a  Net-RID Display Provider to retrieve the details of all existing subscrip- tions  it  created.

NoTE A2.3—As stated,these interfaces are the minimum required.A DSS implementation may include additional interfaces for optimization or functional purposes as long as these minimum interfaces are supported.

(4)After  mapping  and  storing  ISA  summary  information into  the  DSS  Airspace  Representation,the  DSS  shall (DSSO040)not  store  or  otherwise  retain  the precise  geographi- cal  extents  of  the  associated  4-D  volume.

(5)The DSS shall(DSSO050)not allow  more than [NetDSSMaxSubscriptionPerArea]subscriptions  per  USS in a given  area  of the  DSS Airspace Model.

NoTE A2.4—The intention is that Net-RID Display Providers aggregate subscriptions for overlapping display area requests.It is undesirable from a DSS and USS-USS communications eficiency perspective to have a separate DSS subscription for each of a large number of users viewing overlapping areas.

(6)The DSS shall(DSSO060)limit the  duration of sub- scriptions  to  no  more  than  [NetDSSMaxSubscriptionDuration].

NoTE A2.5—Subscriptions should only be established and persisted for areas in which display requests exists.

(7)The  DSS  shall  (DSS0070)be  implemented  in  a  manner that  allows  a USS to  access  any  instance  of a  DSS in  a DSS pool  and  obtain  the  same  results.

NoTE A2.6—If there are multiple production DSS instances in a DSS pool,they nominally are all active and available for use by USSs.

A2.3.2  USS Requirements  Related  to  the  DSS:

A2.3.2.1 The requirements in this  section  apply to Net-RID Service  Providers  or  Net-RID  Display  Providers  interacting with  a  DSS.

(1)A  Net-RID Service  Provider shall(NET0610)make all UAS operations discoverable for Network Remote ID purposes by  means  of one  or  more  ISAs  in  the  DSS  for  the  complete duration  of  each  operation  plus [NetMaxNearRealTimeData- Period].

(2)If a Net-RID Service Provider is unable to make a UAS operation  discoverable  through  the  creation  of  an  ISA  in  the DSS,the  Net-RID  Service  Provider  shall  (NET0620)notify  the operator.

NoTE A2.7—A timely notification is intended,but the manner in which the notification is delivered is at the implementer's discretion and could use electronic or manual methods;therefore,a specific timing requirement for the notification is not specified.An implementer must describe the method(s)used in the product test report.

F3411-22a

a DSS pool.This is necessary to support diverse implementa- tions of DSS instances while ensuring data consistency and interoperability.

A2.5.3 A specific synchronization protocol is not mandated in this  specification.It  is  expected  that  implementers  of the DSS  in  a  DSS  region will  coordinate  and  agree  on  the synchronization  protocol and  other  implementation  details necessary to achieve interoperability in the DSS pool.

A2.5.4 DSS Data Overview—The DSS Airspace Represen- tation (DAR)is the foundational data  structure  for the DSS.  The DAR partitions  airspace  into  discrete  cells  using  some geocoordinate  system.Entities  and  subscriptions  are  mapped onto one or more cells and persisted for their effective time

periods.Consequently,the data for a cell is information about

the  entities  and  subscriptions  (if any)that  intersect  the  cell.

Data that is synchronized between instances of the DSS can logically be understood as a key/value pair where the key is a cell ID and the value is the collection of information about the entities and subscriptions that intersect the cell.A synchroni- zation protocol is used to synchronize this data between DSS instances.

A2.5.5 Data Synchronization Requirements:

A2.5.5.1  When  synchronizing  data,a  DSS  instance  shall (DSSO110)authenticate with other DSS instances in the same region using an industry-standard authentication mechanism.

NoTE A2.10—OAuth 2.0 or later is an example of an industry-standard authentication mechanism by attestation,for those OAuth2 identity providers that require authentication to obtain credentials.

A2.5.5.2 Communication between DSSs in the same region shall(DSS0120)be encrypted using an industry-standard encryption mechanism with a minimum encryption strength of 128 bits.

NoTE A2.11—This requirement is intended to address both integrity and confidentiality  of Remote  ID  data  in transit.TLS  is  an  example  of an industry-standard authenticated encryption mechanism.

A2.5.5.3  DSS  implementations  shall(DSS0130)store  and synchronize the following conceptual data for each DAR cell that has intersecting entities or subscriptions:

(1)The  cell  ID,

(2)For each subscription that intersects the cell:

(a)A unique ID for the subscription;

(b)The owner of the subscription;

(c)The  URL  a  Net-RID Service  Provider  contacts  to inform a Net-RID Display Provider of new,modified,or deleted  ISAs;

(d)Start and end time of the  subscription;

(e)Other  cells  the  subscription  intersects(to  facilitate efficient removal of expired or deleted subscriptions);and

(f)The notification count for the subscription (used by a Net-RID Display Provider to detect missed notifications).

(3)A  Net-RID  Display  Provider  shall  (NET0630)obtain ISA information from the DSS(including creating or maintain- ing an ISA subscription)only for areas in which an end user is currently requesting information by means of the Remote ID Display Applications it services.

A2.4 USS-USS Interfaces

A2.4.1 The requirements in this section address interfaces between Net-RID  Service  Providers  and  Display  Providers. Note  that in  accordance  with 5.5.4.4, authentication and encryption is required for USS-USS interfaces.

A2.4.1.1  A  Net-RID Service Provider shall(NET0710) minimally support the following interfaces for use by Net-RID Display  Providers,in  accordance  with  the  P2P(peer-to-peer) portion of the OpenAPI specification provided in Annex A4:

(1)GET  flights:this  interface  enables  a Net-RID  Display Provider to request the position-related Remote ID data(if any) for UAS operating in one or more volumes.

(2)GET  flight  details:this  interface  enables  a  Net-RID Display Provider to request the additional non-position-related details(for example,UAS ID,UAS type,etc.)for a specific UAS operation.

NoTE A2.8—The separation of position data from other data  serves multiple purposes.It contributes to efficiency by reducing the volume of data transferred;it avoids sharing data that has not been requested;and it provides a mechanism for monitoring for potential abuse cases such as a display  provider  mining information  without an  associated  end  user request.

A2.4.1.2  A  Net-RID  Display  Provider shall(NET0720) query a Net-RID Service Provider for flights only for areas in which end users have requested information by means of the Remote ID Display Applications it services.

A2.4.1.3 Net-RID Display Provider shall (NET0730)mini- mally  support  the  following  interface  for  use  by  Net-RID Service  Providers,in  accordance  with  the  P2P(peer-to-peer) portion of the OpenAPI specification provided in Annex A4:

(1)POST Identification Service Area:this interface is called by a Net-RID Service Provider when the DSS informs it that a Net-RID Display Provider has a subscription for an area intersecting  a new,modified,or  deleted  ISA

NoTE  A2.9—This  interface  is  the  minimum  required.A  USS  may implement additional interfaces for optimization or functional purposes as long as these minimum interfaces are supported.

A2.4.1.4 When a Net_RID Service Provider is informed by the DSS that a Net-RID Display Provider has a subscription for an area intersecting  a new,modified,or deleted ISA,the Net-RID Service Provider shall(NET0740)send the details of the ISA to the Net-RID Display Provider (by invoking the Post Identification Service  Area  interface) in  [NetDpDataResponse95thPercentile]seconds  95%of  the  time and in [NetDpDataResponse99thPercentile]seconds 99%of the time.

(3)For each ISA that intersects the cell:

(a)A unique ID for the Service Area, (b)The owner of the Service Area,

(c)The URL a Net-RID Display Provider contacts to poll the owning Net-RID Service Provider when Remote ID data is needed for the Service Area,

(d)The start and end time of the  Service Area,and

A2.5 DSS-DSS Synchronization

A2.5.1  This  section  is  applicable  only  to  implementers  of the  DSS.

A2.5.2 This section describes requirements for the concep- tual data that must be synchronized between DSS instances in

F3411-22a

(e)Other  cells  the Service  Area  intersects(to  facilitate efficient removal of expired or deleted Service Areas).

A2.6.1.4  DELETE  Subscription—Tests  must  demonstrate that that a subscription can be deleted on any DSS instance and the deletion is reflected on all other DSS instances.

A2.6 DSS Testing

A2.6.1 Approach—DSS implementations are tested as part of a Net-RID Service Provider or Net-RID Display Provider,or both,when a new or modified DSS is being introduced.A test compliance matrix is provided in 10.3 for  USSs.When testing includes  a  new  or  modified  DSS,the  test  program  must demonstrate interoperability at the DSS level.This is accom- plished by including verification of data synchronization in all DSS instances when testing the various DSS interfaces.The following  provides  specific  guidance  for  each  of  the  DSS interfaces(defined  in A2.3.1):

A2.6.1.1 PUT Identification Service Area—Tests must dem- onstrate  that  after  an  ISA  is  created  or  modified,it  can  be retrieved from all DSS instances for the region with consistent results.In addition,the end time for an ISA governs when the DSS  automatically  removes  it  from  the  DSS.Tests  must demonstrate that automatic removal ofISAs occurs on all DSS instances.

A2.6.1.2 DELETE Identification Service Area—Tests must demonstrate that an ISA can be deleted on any DSS instance and the deletion is reflected on all other DSS instances.

A2.6.1.3 PUT Subscription—Tests must demonstrate that a subscription can be created on any instance and notifications for the subscription are triggered when intersecting ISAs are added or modified to all other instances.In addition,the end time for a subscription governs when the DSS automatically removes it from the DSS.Tests must demonstrate that auto- matic removal of subscriptions occurs on all DSS instances.

A2.6.1.5 GET Subscription—Tests must demonstrate that a specific subscription can be retrieved from any DSS instance with consistent results.

A2.6.1.6 GET Subscriptions—Tests must demonstrate that the  complete  set  of subscriptions  in  an  area  for  a Net-RID Display Provider can be retrieved from any DSS instance with consistent  results.

A2.6.2 Test Environment Requirements—DSS implementers shall (DSS0210)provide a persistently  supported test instan- tiation of their DSS implementation,including at least 2 DSS instances,for use by Net-RID Service Providers and Display Providers  for  interoperability  testing.This  test instantiation must use the current deployed version of the implementer's DSS software and be configured to perform DSS-DSS syn- chronization with the test instantiations of other DSS providers in the same region.

A2.6.3  Compliance  Matrix:

A2.6.3.1 The compliance matrix for the DSS is only appli- cable to implementations of a Net-RID  Service  Provider  or Net-RID Display Provider,or both,that introduce a new DSS implementation.In addition to the compliance matrix provided below,implementers  must  demonstrate  compliance  with  the applicable network requirements provided in  10.3 while apply- ing the test guidance provided in A2.6.1.

A2.6.3.2 DSS Compliance Matrix—See Table  A2.1.

TABLE A2.1 DSS Compliance Matrix

| Req ID | Section Reference Compliant(Y/N) Notes |
| --- | --- |
| DSS0010 | A2.3.1 |
| DSS0020 | A2.3.1 |
| DSS0030 | A2.3.1 |
| DSS0040 | A2.3.1 |
| DSS0050t | A2.3.1 |
| DSSO060t | A2.3.1 |
| DSS0070 | A2.3.1 |
| DSS0110 | A2.5.5 |
| DSS0120 | A2.5.5 |
| DSS0130 | A2.5.5 |
| DSS0210 | A2.6.2 |

十Editorially corrected

F3411-22a

A3.TABLES OF VALUES

TABLE A3.1 Broadcast Values

| Value Name |  | Value UoM | Req Ref | Section Ref |
| --- | --- | --- | --- | --- |
| BcMinAvgEIRP BcMaxPeakToAvg BcMinEIRP BCMinUasLocRefreshRate BCMinStaticRefreshRate BCMaxDataAge | Use Use | Table A3.2 dBm 4 dB Table A3.3 dBm 1 Seconds 3 Seconds 1 Seconds | BPWO010 BPWO010 BPWO010 BUR0010 BUR0020 BUR0030 | 5.4.3 5.4.3 5.4.3 5.4.4 5.4.4 5.4.4 |
| Wave Law |  | TABLE A3.2 BCMinAvgEIRP Values BCMinAvgEIRP-Minimum Average Tx EIRP 2.4 GHz Wi-Fi |  |  |
|  |  |  | in Horizontal Plane |  |
|  |  |  | 2.4 GHz Bluetooth | 5 GHz Wi-Fi |
| Type 1(for example,USA) Type 2(for example,China) |  | +13 dBm +11 dBm | +3 dBm +3 dBm | +13 dBm +11 dBm |
| Type 3(for example,EU,Japan, | Korea) | +11 dBm | +3 dBm | +4 dBm |

TABLE A3.3 BCMinEIRP Values

| BCMinEIRP-Minimum Tx EIRP in Horizontal Plane |  |  |  |
| --- | --- | --- | --- |
| Wave Law | 2.4 GHz Wi-Fi | 2.4 GHz Bluetooth | 5 GHz Wi-Fi |
| Type 1(for example,USA) | +11 dBm | +3 dBm | +11 dBm |
| Type 2(for example,China) | +9t dBm | +3 dBm | +9t dBm |
| Type 3(for example,EU,Japan,Korea) | +9t dBm | +3 dBm | +4 dBm |

十Editorially corrected

TABLE A3.4 Network Values

| Value Name | Value | UoM | Req Ref | Section Ref |
| --- | --- | --- | --- | --- |
| NetMinUasLocRefreshFrequency | 1 | Hz | NET0040 | 5.5.2.4 |
| NetMinUasLocRefreshPercentage | 20 7 | Percent km | NET0040 NET0240 NET0250 NET0420 | 5.5.2.4 5.5.4.4 5.5.4.4 5.5.5.10 |
| NetMaxDisplayAreaDiagonal |  |  | NET0430 | 5.5.5.10 |
|  |  |  | NET0440 | 5.5.5.10 |
|  |  |  | NET0480 | 5.5.5.10 |
|  |  |  | NET0490 | 5.5.5.10 |
| NetSpDataResponseTime95thPercentile | 1 | Seconds | NET0260 | 5.5.4.4 |
| NetSpDataResponseTime99thPercentile | 3 | Seconds | NET0260 | 5.5.4.4 |
|  | 60 | Seconds | NET0260 | 5.5.4.4 |
| NetMaxNearRealTmeDataPeriod |  |  | NET0270 NET0450 NET0610 | 5.5.4.4 5.5.5.10 A2.3.2 |
| NetDpMaxDataRetentionPeriod | 86,400 | Seconds | NET0330 | 5.5.4.4 |
| NetDplnitResponse95thPercentile | 6 | Seconds | NET0420 | 5.5.5.10 |
| NetDplnitResponse99thPercentile | 18 | Seconds | NET0420 | 5.5.5.10 |
| NetDpDataReponse95thPercentile | 1 | Seconds | NET0440 | 5.5.5.10 |
| NetDpDataResponse99thPercentile | 3 | Seconds | NET0440 | 5.5.5.10 |
| NetMinSessionLength | 5 | Seconds | NET0440 | 5.5.5.10 |
| NetDpDetalsResponse95thPercentile | 2 | Seconds | NET0460 | 5.5.5.10 |
| NetDpDetailsResponse99thPercentile | 6 | Seconds | NET0460 | 5.5.5.10 |
|  | 2 | km | NET0460 | 5.5.5.10 |
| NetDetailsMaxDisplayAreaDiagonal |  |  | NET0480 | 5.5.5.10 |
|  |  |  | NET0490 | 5.5.5.10 |
| NetMinClusterSize | 15 | Percent | NET0480 | 5.5.5.10 |
| NetMinObfuscationDistance | 300 | m | NET0490 | 5.5.5.10 |
| NetDSSMaxSubscriptionPerArea | 10 | Subscriptions | DSS0050 | A2.3.1 |
| NetDSSMaxSubscriptionDuration | 24 | Hours | DSS0060 | A2.3.1 |

F3411-22a

A4.USS-DSSAND USS-USS OpenAPI YAML DESCRIPTION

A4.1 The API YAML code can be found at the following URL:

https://github.com/uastech/standards/tree/astm_rid_api_2.1

A5.NUMBER REGISTRAR MANAGEMENT POLICY

A5.1  Overview

A5.1.1 The  fields  Specific  Session  ID  Type  and  Specific  Authentication  Method  Type  both  require  a 1-byte  unique  identifier(1-224)0 and 225-255 are reserved.The method  chosen to ensure the identifier is unique is to use a registry  service to register any new types created.Since the available  IDs  are  somewhat  of  a scarce  resource,this  specification  adopts a number management strategy following the Internet  Assigned Number Authority(IANA)“Specification Required” method as outlined in IETF RFC8126,Sec 4.6.The organiza-  tion appointed to manage this registry(registrar)shall be  appointed by the chair of ASTM F38.

A5.2 Registration Requirements

A5.2.1 For an organization to reserve a Specific Session ID Type or Specific Authentication Method Type identifier value (type  identifier),they  (applicant)must  provide  the  following information in the application to the registrar:

A5.2.1.1 A publicly accessible URL to the encoding speci- fication for the identifier being requested from which a skilled software engineer can reliably implement both the transmitter and  receiver.

A5.2.1.2 Organization Name

A5.2.1.3 Organization Mailing Address A5.2.1.4 Primary Contact Name

A5.2.1.5 Primary Contact E-mail Address

A5.3 Registrar Duties

A5.3.1 The registrar shall manage a specification submis- sion  process  for  proposed  type  identifier  assignments  and provide the following services:

A5.3.1.1 Provide a public website URL with the registration requirements and instructions.

A5.3.1.2  Collect  and  maintain  the data  submitted  from

A5.2.

A5.3.1.3  Forward  the(complete)requests for  identifier  to the responsible expert review contact.

A5.3.1.4  Process  review  results  received  from  the  desig- nated  expert(s)(DE)(update published tables  as needed).

A5.3.1.5 Publish the 2 assigned number data tables (Spe- cific Session ID Types,Specific Authentication Method Types) on a public website URL including the following fields:

(1)Assigned Type Identifier

(2)Org  Name

(3)Org  Mailing Address

(4)Contact  Name

(5)E-mail  Address

A5.4 Registration Process

A5.4.1  The  applicant  shall  send  the  required  data  in  the application to the registrar.

A5.4.2 The registrar shall forward the application to the DE. A5.4.3 The DE shall approve or reject the application.

A5.4.4  The registrar  shall  update  the published  tables  as required.

A5.4.5 The registrar shall inform the applicant the result of the process and type identifier assigned as appropriate.

A5.5 Expert Review

A5.5.1  Designated  Expert(s)—The chair  of ASTM  F38 shall appoint and maintain the appointment of a Remote ID Designated  Expert(DE)and  at  least 1  alternate  DE  for  the purpose of reviewing and approving type identifier applica- tions.

A5.5.2 Expert Review Criteria and Process:

A5.5.2.1  The  expert  review  must  approve  based  on  the following criteria:

(1)The proposed specification does not duplicate an exist- ing method.

(2)The proposed specification meets the intent of the field in the F3411 specification (as appropriate for Specific Session ID or Specific Authentication Method).

(3)Type identifiers are efficiently being used and not being “hoarded”or "wasted”.

A5.5.2.2 The expert review shall not deny a request unless one of the three above criteria are not met.If a denial is given, a rationale must be provided.A denial may be appealed,and an alternate DE must adjudicate an appeal.An appeal may not be processed by the DE who processed application under appeal.

A5.5.2.3 Reviews shall be processed by the DE within 4 weeks of receipt of application by the registrar.

APPENDIXES(Nonmandatory Information)

X1.PERFORMANCE CHARACTERISTICS

X1.1 The inherent characteristics of Broadcast and Network Remote  ID  results  in  different  performance  under  varying circumstances.This appendix describes the varying character- istics for each method of performing Remote ID.

X1.2 Broadcast Remote ID Characteristics:

X1.2.1  General—The  general  characteristics  described  in this section apply to all forms of broadcast methods defined in this specification,including Bluetooth 4(BT4),Bluetooth  5 Long  Range(BT5LR),and  Wi-Fi.

X 1.2.1.1 The UA can always deterministically and measur- ably meet the requirement and conform to this specification without  any  dependency  on  externalities  such  as  ground infrastructure.The  broadcast  function  is  always  transmitting whether a receiver is there to "hear"it or not.

X 1.2.1.2 Unlike typical wireless applications that require a "connection,"all of the broadcast methods in this specification use  a one-way “connection-less”broadcast method that does not  require  the  receiver  to  transmit  anything  back  to  the broadcaster.There is no “pairing”or “connecting”to an Access Point using any of the broadcast technologies in this specifi- cation.Therefore  the  amplitude  limitations  of  broadcast  are only  limited  by  the  transmitter's  power  levels  (not  the  re- ceiver).

X 1.2.1.3  That  being  said,the  receiver  antenna  sensitivity and received noise  interference  can  have  an  impact  on  the reception range.

X 1.2.1.4  Broadcast  performs  best  with  the  longest  range where Electromagnetic  Interference(EMI)is  minimal.Con- versely strong EMI will tend to reduce the range of broadcast.

X 1.2.1.5 Broadcast can uniquely satisfy Remote ID require- ments where there is no other means of network coverage and where there is no established federation of network servers and infrastructure.

X1.2.2  Bluetooth  4(BT4)aka  Bluetooth  Legacy:

X1.2.2.1 Industry Standard Approach—BT4 has clear stan- dards(see Bluetooth Core Specification 5.0,Volume 6,Part B, Sections 2.1 and 2.3.1)outlined and implementations existing for many years.

X 1.2.2.2 Technology—The technique the standard uses is called"Connectionless  Advertisements."These  advertisements continually broadcast without the need  of a receiver to  ac- knowledge.It is a similar mechanism to when a device is in “pairing mode”and  a receiver will  list the  devices ready to pair.The  receiver  can see  these devices  because  they are broadcasting pairing  advertisements.These advertisements occur  on  three  dedicated  channels  (of  40)called "beacon channels"that are strategically on the 2.4 GHz spectrum away where Wi-Fi traffic most commonly occurs.

X 1.2.2.3 Receiver Support—BT4 is widely supported on the receiver  side hardware  and  software.Both  iOS  and  Android have to,with widely used APIs,enable software development of receiving applications without the need of auxiliary hard- ware or dongles.

X 1.2.2.4  Range—Because  of  the  speed  being  limited  to 1 Mb,the range is often better than expected (for a given power level).It  has  been  tested  to  have  about  400m  in  a  rural environment.Lots of RF interference could reduce this range. For the power levels outlined in this  specification,BT4 will generally have a shorter range than Wi-Fi and Bluetooth 5 at the power levels defined in this specification.

X1.2.2.5 Limited Payload (26 Bytes)—Because of the lim- ited payload size,the full complement of broadcast Remote ID data cannot  fit in a  single message.Consequently,this  speci- fication  uses  multiple  small  messages  to  transmit  the  data. Since the data is transmitted over multiple messages,authen- tication mechanisms are limited to verification of static infor- mation transmitted (to maintain coherency between authenti- cation data and data to be validated).

X1.2.2.6 Transmitter Hardware—Transmitter  hardware modules  are  available  for  commodity  prices.Such  hardware modules  have  a  programmable  microprocessor  with serial (UART)interfaces  for  getting  telemetry  data  from  the  UA which is retransmitted through the connected antenna.

X 1.2.2.7 Bluetooth Transmitters do not generally exist on UAs,so they would have to be retrofitted or engineered into

future UAS products to use this technology X1.2.3 Bluetooth  5 Long Range(BT5LR):

X1.2.3.1 Industry Standard Approach—Although BT5LR is relatively new,clear standards exist on how to implement and such standards are being widely adopted.

X 1.2.3.2  Technology—B5LR  incorporates  two  new  key technologies introduced in the Bluetooth 5.0 specification:

(1)A coded phy that encodes the data to operate at a much slower rate than BT4(125 kbps)and it also includes Forward Error  Correction(FEC).This  slower  rate  coupled  with  FEC drastically increases the potential range.

(2)Advertising Extensions that allow small "Pointer Bea- cons"to  tell  the  receiver  where(on  the  other  40  channels)to read the larger payload.This allows larger(255 byte)payloads and a wider distribution of traffic over 40 channels.

X 1.2.3.3 Range—Given the same environment,BT5LR generally gets about 4 times that of BT4.In rural environments, tests  are  showing  over  1  km.Lots  of RF  interference  could reduce this range.

X 1.2.3.4  Large  Payload—Since  BT5LR  allows  for  large (255 byte)payloads,it enables more data architecture flexibil- ity allows a full set of messages to be sent in a single packet. By doing this,if desired,a complementary  authentication payload  can  be  sent  in  full  coherency  with  the  rest  of the payload,therefore allowing the entire payload to be verified authentic.

F3411-22a

X 1.2.3.5  Receiver  Support—Although  BT5LR  is  widely enabled on the transmitter side,it is still new and only a few (smartphone)products support it on the receiver side.As of this writing,we have received information that newer smartphones from Samsung(Galaxy S10+),Xiaomi(Mi 9),and Huawei (Mate  20  Pro)support  BT5LR.Also,long  range  mode  is  an optional mode of BT5 so some products that claim BT5 may or may  not  have  adopted  the  Long  Range  mode.However, BT5LR transmitters are compatible with BT4 receivers as long as  the  BT5  radios  transmit  in  Legacy  Mode.Therefore  the standard requires that those using BT5 must perform both Long Range mode and BT4(Legacy)Mode to achieve both universal compatibility and the benefits of Long Range.Although BT5LR is not yet widely enabled in current smartphones,if one desires to receive the long range broadcasts,this can be achieved by adding a “dongle”attachment to a smartphone or by using some fixed ground station IOT device,or laptop to which a USB receiver could be connected.

X1.2.3.6 Transmitter Hardware—A wide variety of trans- mitter hardware exists to support BT5LR at commodity prices. Some  modules  even  have  an  integrated  power  amplifier  to extend  the  range.Such  hardware  modules  have  a  program- mable microprocessor with serial interfaces for getting telem- etry  data  from  the  UA  which  is  retransmitted  through  the connected  antenna

X 1.2.3.7 As with BT4,it does not commonly exist on UAs today,therefore  it  would  have  to  either  be  retrofitted  onto existing UAs or designed into newer products.

X1.2.4 Wi-Fi Aware:15

X 1.2.4.1 Industry Standard—The“Wi-Fi Aware”15,11 proto- col used in this specification was developed by a workgroup of many industry stakeholders within the Wi-Fi Alliance.

X1.2.4.2  Technology—Wi-Fi  Aware¹⁵is  a  technique  that uses  Wi-Fi  radios  to  send  broadcast  messages  using  IEEE standard “public action frames”formatted in a standard way to transmit arbitrary data.

X 1.2.4.3  Range—The  standard  prescribes  higher  power numbers  for  Wi-Fi  than  Bluetooth  because  of  the  higher transmit speed(6Mbps)and the wide availability of radios that support this higher power level.In rural environments,tests are showing over 1 km(at 14 dbm),and for typical power levels in US and EU the expected ranges are more than 2 km at 20 dBm in  EU,and  more  than  4  km  at  26dBm  in  US.Lots  of  RF interference could reduce this range.

X 1.2.4.4  Large  Payload—By  having  a  large(255  byte) payload,all messages can be sent as a grouped set that allows more authentication options(similar to BT5LR).

X 1.2.4.5 Receiver Support—Wi-Fi Aware¹⁵is a bit new in terms  of  direct  operating system  and smartphone  support. Android 8(Oreo)released in 2017 is the first smartphone OS to support Wi-Fi Aware.¹⁵Almost all mainstream mobile Wi-Fi chipset manufacturers have supported Wi-Fi Aware.¹⁵Mobile

phone manufacturers could support Wi-Fi Aware¹⁵as long as they  complete  the  necessary  software  adaptation.As  of this writing,newer  smartphones  by  Google(Pixel  2,3),Xiaomi(Mi 8,Mi9,Mix3),and  most  recently,Samsung(Galaxy  Note 10) all  support  Wi-Fi  Aware¹⁵according  to  published  specifica- tions.iOS does not yet support Wi-Fi Aware.⁵

X1.2.4.6 Transmitter Hardware—Since many modern con- sumer UAS include Wi-Fi radios,most of them will be able to enable  this  with  simply  applying  a  firmware  update  and retrofitting hardware will not be necessary.

X1.3 Network Remote ID Characteristics

X1.3.1 Industry  Standard—The protocols  created  in  this specification are all built upon modern RESTful web services protocols that have been tried and tested for many years.

X1.3.2 Technology—Since the protocols are written into the standard to be above "layer 4,"any kind of “layer 2,3”network media(Cellular,Private  Network,Wi-Fi,Satellite)that meets the performance criteria could potentially be used.Most of the group sees cellular networks as the early enablers to provide this  VLL(low  altitude)network  infrastructure.

X1.3.3  Infrastructure—In addition to a network infrastructure,a server/service infrastructure needs to be turned on to provide the required services of Network Remote ID. Much of this specification describes the requirements of such services.These  services  will  all  leverage  the  Internet  as  the common carrier between federation members.As such,in the event  of dependent  server  or  network  outages,Network  Re- mote ID could experience outages as well.

X1.3.4 Range—The only range limitation that may exist for Network Remote ID is the UAS access to the network.As long as the UAS can reach a network(such as cell service),and as long as the intended receiver can reach the network,then the range from the transmitter to the receiver has no limitation.

X1.3.5 Receiver Support—Since nearly all smartphones en-  joy the connectivity to the Internet,receiver support is broad  and consistent across Android and iOS and universally across  hardware platforms.Also,operators of conventional computer  platforms  (laptops,desktops,etc.)that  are  network  connected  would universally be able to participate in the network as well. 	X1.3.6  Transmitter  Hardware—A  number  of  transmitter media and topologies could be used to connect to the network.

For example,a UA could have a cell modem on the aircraft itself,or  it  could  solely  communicate  with  a  ground  station which  has  a  built-in  cell  phone  (as  many  do)which  could provide this network connectivity.

15A trademark of Wi-Fi  Alliance,10900-B  Stonelake  Boulevard,Suite  126, Austin,TX 78759.

X1.3.7 Coverage—Support areas for Network Remote ID is generally  in  areas  with  cellular  network  coverage(unless another network technology is used).Near highways and urban areas and generally where people are,cell network coverage is generally available.If there  are areas  of“spotty”network coverage,this  specification  specifies  ways  of  interpolating position reports even if the UA occasionally ventures to the edge of network coverage.

F3411-22a

X2.LIST OF SUBCOMMITTEE PARTICIPANTS AND CONTRIBUTORS

TABLE X2.1 List of Subcommittee Participants and Contributors

| Name | Organization |
| --- | --- |
| Hanson,Richard Borda,Fred Coleman,Sean Lamprecht,Andreas Voß,Thomas LesterEdward (Ted) Shestopalov,Andrea Hegranes,Jon Cassidy,Sean Champagne,Robert Roth,Robert Sousa,Andrew Ganjoo,Amit Murphy,David Sugahara,Kenji Kenul,Philip(Chair,ASTM F38 UAS Committee) Mikolajewski,Mary(ASTM F38 Staff Manager) Daly,Brian Huffstutler,Tommy Musgrove,Charles Hedden,Carole Baum,Michael Card,Stu Wiethuechter,Adam Doty,James Caina,Javier Schulman,Brendan Brchl,Lukas Bern,Evelina Chen,Bin "David" Ghimire,Ritesh Hendrickson,Adam Hinaman,Arthur Liang,Diana May,Rick Sadeghi,Sam Saunders-Hodge,Sabrina SegersRobert Curdy,Benoit MessinaDavid Nasman,Kevin McCallister,David Moskowitz,Robert Cox,Gabriel(Workgroup Chair) Davis,Mark Edward Friis,Soren Mo,Stan Takei,Jun Minami,Masaki Elliott,Ken Bender,Walter Silbermann,Joshua Olson,Gregory Sehgal,Ajay(Vice Chair,ASTM F38 UAS Committee) Gunnarson,Tom Elefant,Andrew McGinnis,Edward Ryker,Kyle Teer,Caden Capuder,Lawrence Jessen,lan Maroney,David Lacher,Andrew Namduri,Kamesh Thurling,Andrew Driver,Ted Le BailManuel Bullock,Gary Collins,Michael Howard,Larry Pierce,Aaron | Academy of Model Aeronautics Aerial Innovation AirMap AirMap AirMap AiRXOS,part of GE Aviation AiRXOS,part of GE Aviation Aloft Technologies Amazon Amazon Amazon American Robotics ANRA Technologies ANRA Technologies Ariascend ASTM International ASTM International AT&T AT&T AT&T Aviation Week Aviators Code Initiative AX Enterprise AX Enterprise Collins DJI DJI Dronetag FAA FAA FAA FAA FAA FAA FAA FAA FAA FAA FOCA FPV Freedom Coalition Hidden Level Horizon Hobby HTT Consulting Intel Intel Intel Intel Intel IPA DADC,Japan Jetcraft JHU Applied Physics Laboratory JHU Applied Physics Laboratory KBR KBR Kitty Hawk Aviation &Aerospace Kittyhawk.io Mississippi State University RFRL Mississippi State University RFRL Mississippi State University RFRL MIT Lincoln Laboratory MIT Lincoln Laboratory MITRE Noblis University of North Texas NUAIR One Sky Systems Parrot Pierce Aerospace Pierce Aerospace Pierce Aerospace Pierce Aerospace |

TABLE X2.1  Continued

| Name | Organization |
| --- | --- |
| Van Duren,Drew | Qualcomm |
| Jennings,Richard | REJ Aviation Services |
| Brown,Simon | RelmaTech |
| Hall,Philip | RelmaTech |
| Bachrach,Abraham | Skydio |
| Groves,Brendan | Skydio |
| Hochdorf,Eyal | Skydio |
| Player,Jennifer | Skydio |
| Bennington,Jeremy | Spirent |
| Morrison,Adam | Streamline Designs |
| Deeds,Greg | Technology Exploration Group |
| Ruff,Nathan | UASidekick |
| Ramsey,Christian | uAvionix Corporation |
| Belaus,Greg | Uber |
| Prevot,Tom | Uber |
| Wei,Bogu | Uber |
| Broux,Jan | Unifly |
| Huenaerts,Laurent | Unifly |
| Fanelli,Matt | Skyward,A verizon company |
| Lincoln,David | Skyward,A verizon company |
| O'Neill,Bri | Skyward,A verizon company |
| Blanks,Mark | Virginia Tech |
| Fox,Luke | WhiteFox Defense Technologies |
| Hodgens,Ryan | WhiteFox Defense Technologies |
| Peterson,Zachary,PhD | WhiteFox Defense Technologies |
| Florn,Alexandra | Wing |
| Glasgow,Mike(Network Lead) | Wing |
| Jackman,Chris | Wing |
| Negron,Reinaldo (Data Lead) | Wing |
| Pelletier,Benjamin | Wing |
| Teeling,Sean | Wing |
| Woodworth,Adam | Wing |
| Clark,Steve | WISekey |
| Wurzbach,Jeffrey | Wurzbach Electronics,Inc. |
| Reddy,Hersh | Zipline International |
| Wolf,Harrison | Zipine International |

X3.BACKGROUND INFORMATION

X3.1  In  2017,the  Federal  Aviation  Administration(FAA)  chartered the Unmanned Aircraft Systems(UAS)Identification  (ID)and Tracking Aviation Rulemaking Committee(ARC)  (UAS-ID  ARC)to  provide  recommendations  to  the  FAA  regarding technologies available for remote identification and  tracking  of UAS.In  its  final  report,⁶the ARC recommended  two methods for remote identification,those being “broadcast” and  "network.”"Broadcast"would  require  UAS  to  transmit  information without bi-directional communication with a re-  ceiver.“Network”would require UAS to  communicate  infor-  mation to a network,whether space or terrestrial.

X3.2  In  2018,the  ASTM  F38  Committee  on  Unmanned Aircraft  System  spearheaded  the  effort  to  define  minimum performance standards for remote identification of UAS.Work- ing  in  conjunction  with  the  FAA,members  of  the  ASTM  Subcommittee  F38.02  on  Flight  Operations  began  work  on creating the standards for Remote ID and Tracking.During the summer of 2018,the group completed the Terms of Reference (TOR)that would guide  development  of the  standard.At  the suggestion  of the  FAA,the  Subcommittee  created  use  cases that  aligned  with  the  recommendations  from  the  ARC.The FAA also gave guidance that both methods of Remote ID and

16FAA,Unmanned Aircraft  Systems(UAS)Identification(ID)and  Tracking Aviation  Rulemaking Committee(ARC)(UAS-ID ARC)Recommendations Final

Report,https://www.faa.gov/news/updates/?newsId=89404.

tracking would need to be compatible with commonly carried handheld devices such as cellphones.

X3.3 Use Cases

X3.3.1  The  use  cases  fell  into  two  broad  categories.The first  addressed  security  and  public  safety  needs  of the  law enforcement,homeland defense,and national security commu- nities for the remote identification and tracking of UAS.The second category addressed public access to remote identifica- tion information as a secondary objective.

X3.3.2 The primary high-level “representative”and unoffi- cial use  cases  considered  including the  following  and  were inspired  by  the  UAS-ID  ARC  and  discussions  within  the workgroup.

X3.3.3 Public Safety  &Public  Concerns—The  UA  unique identifier  should  be  available  to  the  public  and  the  public should be able to read this identifier during flight so that they could request assistance from public safety officials.

X3.3.4 Area Threat Assessment:

X3.3.4.1 Public safety officials are responding to a citizen report of a“suspicious UA”that is currently operating.

X3.3.4.2  Public  safety  officials  are  providing  security  to  critical infrastructure or venue and spot a “suspicious UA.” They  need  to identify  the operator,make  a threat  determination,and  respond  accordingly.

X3.3.6.4 A private industrial company(perhaps a refinery or some other location considered “critical infrastructure”)would like to patrol the security of the area including the low altitude airspace.They would like to deploy receivers to alert(private) security personnel of incursions over their sensitive location and inform public officials to address the problem.

X3.3.6.5 Airport security would like to detect unauthorized incursions into their low altitude airspace using a network of fixed base receivers.

X3.3.6.6  There  is  a  VIP  event  in  an  area  and  security personnel  use either  a  mobile application  or  fixed  based receivers to identify UAS in that area.

X3.3.6.7 UAS operating under UTM would like to detect other  UAS  nearby  whether  they  are  operating  under:(1)the same USS,(2)another USS,or(3)outside of UTM.

X3.3.6.8  A  UAS  operating  under  BVLOS  would  like  to avoid a path conflict with another UAS.

X3.3.7 All of these inputs,use cases,reviews,and feedback went into consideration in the development of this specifica- tion.

X3.3.5 This category addresses the need to have dynamic and active awareness of UAS near heightened awareness areas. These  areas  could  include:airports,heliports,prisons,military installations,nuclear  facilities,large  stadiums,and  other  criti- cal infrastructure locations where a UAS could potentially pose an imminent threat to public safety.

X3.3.6 Of course,many other derivative use cases could be expanded from this list,such as the following:

X3.3.6.1 Public safety official receives a complaint about a UAS that may or may not be flying out of compliance and the official seeks to identify the UAS,Remote Pilot,contact information,and  pilot  location  using  a  standard  handheld device.

X3.3.6.2 Someone in the general public sees a UAS oper- ating in a way that causes them concern,but eventually departs the  area.They would  like to report the UAS(ID)to  a public safety official (using a smartphone).

X3.3.6.3 Someone in the general public sees a UAS fly over their area and wants to identify the UAS.

ASTM Intermnational takes no position respecting  the  validity of any patent rights  asserted in  connection  with  any  item mentioned in  this  standard.Users  of this  standard  are  expressly  advised  that  determination  of the  validity  of any such  patent rights,and  the  risk of  infringement  of  such rights,are  entirely  their  own responsibility.

This standard is subject to revision at any time by the responsible technical committee and must be reviewed every five years and if not revised,either reapproved or withdrawn. Your comments  are  invited  either for revision  of this  standard  or for aditional standards and  should  be  addressed  to  ASTM  Intermational  Headquarters. Your  comments  wil  receive  careful  consideration  at  a  meeting  of  the responsible  technical  committee,which  you  may  attend.If you  feel  that  your  comments  have  not  received  a  fair  hearing  you  should make  your  views  known  to  the  ASTM  Committee  on  Standards,at  the  address  shown  below.

This  standard  is copyrighted  by  ASTM International,100  Barr  Harbor  Drive,PO Box C700,West Conshohocken,PA 19428-2959, United  States.Individual  reprints  (single  or  multiple copies)of  this standard may  be  obtained  by  contacting  ASTM  at  the  above address  or at  610-832-9585(phone),610-832-9555(fax),or  service  @astm.org(e-mail);or  through  the  ASTM  website (www.astm.org).Permission rights to photocopy the standard may also be secured from the  Copyright Clearance  Center,222 Rosewood Drive,Danvers,MA  01923,Tel:(978)646-2600;http://www.copyright.com/